IBM 1620 Jr.

Diagnostic Manual

Version 0.9

Introduction

The IBM 1620 was always shipped with a full set of diagnostic programs.  These programs were typically used by the IBM Customer Engineer to check the correct operation of the machine and to diagnose / isolate / exercise failures when they occurred.  They cover every aspect of the machine from console switches & lights, core memory, and instruction execution to operation of peripherals.  A properly maintained IBM 1620 rarely failed, but it did happen.  The diagnostics were also used when the computer was first installed to verify that it was fully operational.

While the hardware of the IBM 1620 Jr. is several orders of magnitude more reliable than the original 1960’s IBM 1620, the diagnostic programs are still very useful to validate the simulation software in the main unit, the typewriter, and the card reader/punch device.  Whenever changes are made to the simulators, the diagnostics should be run.

The Computer History Museum has in its collection, most of the manuals for the IBM 1620 Model 1 Diagnostics and a few paper tape images of the basic ones.  Fortunately, for the diagnostics without binary images, the manuals include a simplified machine language listing of each program.  One notable exception is the CU06/DT106 floating point diagnostic which is completely missing from the collection.  As a replacement, a volunteer, John M. Bohn, wrote an SPS program (FP01) which thoroughly tests all of the 1620’s floating point instructions.  In addition, the CU02 Error Check diagnostic only tests the memory addressing register (MAR) subset of the IBM 1620’s “Check Stop” conditions and depends on an internal maintenance switch, CE #9, which changed the computer’s behavior.  For the IBM 1620 Jr. it was replaced with a new, comprehensive error checking program (CS01).

IBM supplied all of the diagnostics on paper tape, punched cards, and the customer engineering disk pack.  For the IBM 1620 Jr. all of the diagnostic programs have been “built into” the simulator for ease of use.  When the IBM 1620 Jr. is powered off, pressing the RESET button loads into simulated core memory one of the diagnostic programs based on the setting of the PROGRAM SWITCHES.  When the machine is turned on, pressing the START button will run the program.

Here are the programs available:

| PS 1 | PS 2 | PS 3 | PS 4 | Name | Description |
| --- | --- | --- | --- | --- | --- |
| off | off | off | off | PowerOf2 | Power of 2 Calculator Demo Program |
| off | off | off | on | CU00 | Console Diagnostic |
| off | off | on | off | CU01 | General Op Codes Diagnostic |
| off | off | on | on | CU05 | Special Instructions Diagnostic |
| off | on | off | off | FP01 | Floating Point Diagnostic |
| off | on | off | on | CU03 | Indirect Addressing Diagnostic |
| off | on | on | off | CS01 | Check Stops Diagnostic |
| off | on | on | on | DX05L | Core Storage 20K Low Diagnostic |
| on | off | off | off | DX05H | Core Storage 20K High Diagnostic |
| on | off | off | on | CU04 | Additional Core Diagnostic |
| on | off | on | off | DX03 | Typewriter Diagnostic |
| on | off | on | on | IO02 | Card I/O Diagnostic |
| on | on | off | off | IO03 | Card I/O Reliability Diagnostic |

The following sections describe all of the diagnostic programs and how to run them.

Lights and Switches Tests

Lights and Switches Tests

Built into the IBM 1620 Jr. simulator is a way to manually test all of the lights and switches on the front panel.  This is the IBM 1620 Jr.’s version of a “Lamp Test” switch found on other computers.  The IBM 1620 did not have this feature, but it is a useful capability for maintenance, so it was added to the IBM 1620 Jr.  Unlike the IBM 1620 diagnostic test programs, these tests do not involve executing any IBM 1620 code.

There are four different tests available.  They are activated when the machine is powered off, the left-most (unlabeled) toggle switch is on, and one PROGRAM SWITCH is turned on.

PROGRAM SWITCH 1:  This is the actual “Lights and Switches” test.  It begins with all wired lights turned on.  This tests all of the lights.  As each switch is turned on or button is pressed, one or more lights are turned off.  For the toggle switches [not the PROGRAM SWITCHes], the light(s) above it are turned off when the switch is on.  The left-most toggle switches and the PROGRAM SWITCHes are checked by selecting the various tests.  The button / light correspondence is:

RESET:  POWER ON, POWER READY, THERMAL lights

DISPLAY MAR: 	Pressing this button tests the large MEMORY ADDRESS REGISTER DISPLAY SELECTOR rotary switch.  The HUNDREDS row of the MEMORY ADDRESS REGISTER displays a binary value representing the position of the knob as it is rotated, where:  OR-1 = 0, OR-2 = 1, OR-3 = 2, …, IR-2 = 11

SAVE:  SAVE light

INSERT:  INSERT light

RELEASE:  PUNCH NO FEED, READER NO FEED lights

START:  AUTOMATIC light

STOP / SIE:  MANUAL light

INSTANT STOP / SCE:  CHECK STOP light

PROGRAM SWITCH 2:  This test displays the full LED brightness range.  Each column of lights in the upper front panel displays a different brightness level from off in the left-most column to fully on in the right-most column.  The columns in-between are 36 of the 254 intermediate brightness levels available.

PROGRAM SWITCH 3:  This “snake” demo moves multiple lights across the upper and lower front panels.  The two lights in the upper panels move randomly.  The two lights in the lower panels move back and forth across the available wired lights.  The speed of animation is controlled by the MEMORY ADDRESS REGISTER DISPLAY SELECTOR rotary switch, where PR-3 is the fastest and OR-4 is the slowest.

PROGRAM SWITCH 4:  This one is just for fun.  It lights “IBM 1620” in the upper front panels.

One thing to note about the IBM 1620 Jr.’s front panel – not all of the lights are wired and therefore they cannot be turned on.  This is due to a combination of design decisions made when Jr. was built.  None of the inactive lights were used on a real IBM 1620 Model 1 Level F machine, so it does not affect correct operation of the IBM 1620 Jr.  However, the unlit lights are noticeable in the “lights and switches” tests.  The inactive lights are:

INSTRUCTION AND EXECUTE CYCLE:  E-24, E-25

CONTROL GATES:  SET 00080, VRC GATE, END COMPR

INPUT-OUTPUT:  row 1 light 3, row 3 light 1, row 5 light 1

DIGIT REGISTER:  row 1 light 2, row 2 light 2

Lower Left Panel:  row 1 light 1, row 1 light 5, row 2 light 1, row 3 light 1, MBR-E CHK, MBR-O CHK

Status Lights:  light 8, light 11, light 13

CU00 - Console Diagnostic

//==============================================================================

//

// CU00 - Console Diagnostic

//

// Program Switch settings:

//

//   PS1: not used

//   PS2: not used

//   PS3: not used

//   PS4: not used

//

// Check switches settings:

//

//   DISK I/O - STOP

//   PARITY   - STOP

//   I/O      - STOP

//   O'FLOW   - STOP

//

// Start addresses:

//

//   00000 - Full test

//

// Directions:

//

//   1. Load CC diagnostic

//   2. Press RESET

//   3. Press START

//   4. After printing the diagnostic program, the machine will halt with the

//      MANUAL light on

//   5. Verify the register contents:

//      OPERATION REGISTER - 48

//      MULTIPLIER - 0

//      SENSE AND BRANCH - 0

//      MEMORY BUFFER REGISTER - 22

//      MEMORY DATA REGISTER - 2

//      DIGIT REGISTER - 22

//      MEMORY ADDRESS REGISTER - 11111

//   6. Press SAVE

//   7. Verify the MEMORY ADDRESS REGISTER is 11112

//   8. Turn the MEMORY ADDRESS REGISER DISPLAY SELECTOR, press DISPLAY MAR, and

//      verity the MAR register contents:

//      IR-1 - 11112

//      IR-2 - 00000

//      OR-1 - 02222

//      OR-2 - 04444

//      OR-3 - 04444

//      PR-1 - 11112

//      PR-2 - 00000

//      PR-3 - 00000

//   9. Press INSERT

//  10. Type 4911112

//  11. Press RELEASE-START

//  12. Verify AUTOMATIC light is on and the MANUAL light is off

//  13. Type ABCDEFGHIJKLMNOPQRSTUVWXYZ <record mark>

//  14. Press RELEASE

//  15. Press START

//  16. After printing the alphabet, the machine will wait for alphameric input

//  17. Verify the AUTOMATIC light is on and the SAVE light is off

//  18. Verify the MEMORY ADDRESS REGISTER is 11123

//  19. Press RELEASE

//  20. Verify the AUTOMATIC light is off and the MANUAL light is on

//

//==============================================================================

Sample Output – CU00

CU01 - General Op Codes Diagnostic

//==============================================================================

//

// CU01 - General Op Codes Diagnostic

//

// Program Switch settings:

//

//   PS1:   ON - Bypass error type out

//         OFF - Type out routine number on error

//   PS2:   ON - Loop in routine

//         OFF - Continue to next routine

//   PS3:   ON - Stop on error

//         OFF - Do not stop on error, continue

//   PS4:   ON - Repeat test CU01

//         OFF - Run test CU01 once

//

// Check switches settings:

//

//   DISK I/O - STOP

//   PARITY   - STOP

//   I/O      - STOP

//   O'FLOW   - PROGRAM

//

// Start addresses:

//

//   00828 - Full test w/o automatic divide

//   14004 - Full test w/ automatic divide

//

// Directions:

//

//   1. Load CU01 diagnostic

//   2. A tab stop is automatically set at column 10

//   3. Press START

//   4. Load blank cards in card punch

//   5. Press START

//   6. After 1620 stops, move card deck from punch to reader

//   7. Press START

//

//==============================================================================

Sample Output – CU01

CU05 - Special Instructions Diagnostic

//==============================================================================

//

// CU05 - Special Instructions Diagnostic

//

// Program Switch settings:

//

//   PS1:   ON - Bypass error type out

//         OFF - Type out routine number on error

//   PS2:   ON - Loop in routine

//         OFF - Continue to next routine

//   PS3:   ON - Stop on error

//         OFF - Do not stop on error, continue

//   PS4:   ON - Repeat test CU05

//         OFF - Run test CU05 once

//

// Check switches settings:

//

//   DISK I/O - STOP

//   PARITY   - STOP

//   I/O      - STOP

//   O'FLOW   - PROGRAM

//

// Start addresses:

//

//   00828 - Full test

//

// Directions:

//

//   1. Load CU05 diagnostic

//   2. Press START

//   3. Press START

//

//==============================================================================

Sample Output – CU05

FP01 - Floating Point Instructions Diagnostic

//==============================================================================

//

// FP01 - Floating Point Instructions Diagnostic

//

// Program Switch settings:

//

//   PS1:   ON - Bypass error type out

//         OFF - Type out routine number on error

//   PS2:   ON - Loop in routine

//         OFF - Continue to next routine

//   PS3:   ON - Stop on error

//         OFF - Do not stop on error, continue

//   PS4:  not used

//

// Check switches settings:

//

//   DISK I/O - STOP

//   PARITY   - STOP

//   I/O      - STOP

//   O'FLOW   - PROGRAM

//

// Start addresses:

//

//   00402 - Full test

//

// Directions:

//

//   1. Load FP01 diagnostic

//   2. Press START

//

//==============================================================================

Sample Output – FP01

= = = = = = = = = = = = = = = = = = = =

To be supplied when Floating Point

is implemented in the Simulator.

= = = = = = = = = = = = = = = = = = = =

IBM 1620 Jr. SPS Assembler (v0.99)    Source: fp10-main.sps,...    Assembled: 8/23/2020 @ 21:52

------ 1: fp10-main.sps --------------------------------------------------

1:1                           * ************************************

1:2                           * IBM 1620 FLOATING PONT PACKAGE TEST

1:3                           * ************************************

1:4                           *

1:5                           * -----------------------------------

1:6                           * RUNNING IN 1620 SIMH

1:7                           *    SET CPU FP

1:8                           *    SET CPU DIV

1:9                           *    SET CPU IA

1:10                          *    SET CPU MOD1

1:11                          *    SET CPU 60K

1:12                          *

1:13                          *  TURN ARITHMETIC CHECK STOP SWITCH OFF

1:14                          *    DEPOSIT ARSTOP 0

1:15                          *

1:16                          *  SENSE SWITCHES OFF

1:17                          *    DEPOSIT SW1 0

1:18                          *    DEPOSIT SW2 0

1:19                          *    DEPOSIT SW3 0

1:20                          *    DEPOSIT SW4 0

1:21                          *

1:22                          * -----------------------------------

1:23                          * -----------------------------------

1:24                          * SENSE SWITCH SETTINGS

1:25                          *    SW1 ON  = DO NOT PRINT ERROR

1:26                          *    SW2 ON  = REPEAT TEST

1:27                          *    SW3 ON  = HALT ON ERROR

1:28                          *

1:29                          *

1:30                          * ==========================================================

1:31                          * REF  1. 227-5630-1 IBM 1620 FLOATING POINT FEATURE CE MANUAL 1962

1:32                          *      2. G26-5595-0 AUTOMATIC FLOATING POINT OPERATIONS SEP61

1:33                          *

1:34                          * P = P ADDRESS

1:35                          * Q = Q ADDRESS

1:36                          * PF (MP) = CONTENTS OF P

1:37                          * QF (MQ) = CONTENTS OF Q

1:38                          * PE (EP) = EXPONENT OF THE FIELD AT THE P ADDRESS

1:39                          * QE (EQ) = EXPONENT OF THE FIELD AT THE Q ADDRESS

1:40                          * L = NUMBER OF DIGITS IN THE MANTISSA

1:41                          * D = PE-QE (EP-EQ)

1:42                          *

1:43                          * REF 2 USES MP, MQ, AND EP, EQ

1:44                          *

1:45                          * ==========================================================

1:46                          * SUMMARY

1:47                          *  DATA

1:48                          *      FLOATING SHIFT RIGHT (FSR-08)

1:49                          *      FLOATING SHIFT LEFT (FSL-05)

1:50                          *      TRANSMIT FLOATING (TFL-06)

1:51                          *

1:52                          *      BRANCH AND TRANSMIT FLOATING (BTFL-07)

1:53                          *

1:54                          * ARITHMETIC - FROM REF 1 FLOATING POINT ARITHMETIC EXAMPLES

1:55                          *              FLOATING POINT ARITHMETIC COMMAND RESULTS

1:56                          *              ARE VERIFIED WITH FIXED POINT COMMANDS

1:57                          * ----------------------------------------------------------

1:58                          *   FLOATING ADD (FADD-O1)

1:59                          *     EXAMPLES 1-5 EXPONENT CONFIGURATION

1:60                          *     EXAMPLES 6-11 FRACTION (MANTISSA) CONFIGURATION

1:61                          *

1:62                          *       EXAMPLE 1 PE = QE / D = 0

1:63                          *       EXAMPLE 2 (PE GT QE) SCAN Q -- FRACTION (MANTISSA) ADD-- D LT L

1:64                          *                   QF SHIFTED D (PE-QE) RIGHT TO ALIGN DECIMAL POINTS

1:65                          *       EXAMPLE 3 (PE GT QE) SCAN Q -- FP EXIT -- D GT L

1:66                          *                  QE IS TOO SMALL-- Q DISCARDED-RESULT IS P

1:67                          *       EXAMPLE 4 (PE LT QE) EXP TRANSMIT SCAN P - FRACTION (MANTISSA) ADD-

1:68                          *                 D LT L - PF SHIFTED D (PE-QE) RIGHT TO ALIGN DECIMALS

1:69                          *       EXAMPLE 5 (PE LT QE) EXP TRANSMIT SCAN P -- FP EXIT -- D GT L

1:70                          *                  PE IS TOO SMALL-- P DISCARDED-RESULT IS Q

1:71                          *       EXAMPLE 6 NO NORMALIZING REQUIRED---NO CARRY OUT

1:72                          *       EXAMPLE 7 NORMALIZING SHIFT RIGHT REQUIRED -- CARRY OUT

1:73                          *                 EXPONENT ADJUST

1:74                          *       EXAMPLE 8 NORMALIZING SHIFT RIGHT REQUIRED -- MACHINE INFINITY

1:75                          *                 9...9 99 - CARRY OUT CAUSES EXPONENT OVERFLOW

1:76                          *       EXAMPLE 9 ZERO FRACTION (MANTISSA) RESULT -- MACHINE ZERO 0...0 -99

1:77                          *       EXAMPLE 10 NORMALIZING SHIFT LEFT -- SHIFT LEFT -- ZERO FILL

1:78                          *                  EXPONENT ADJUST

1:79                          *       EXAMPLE 11 NORMALIZING SHIFT LEFT -- EXPONENT OVERFLOW

1:80                          *                  MACHINE ZERO

1:81                          *

1:82                          *   FLOATING SUBTRACT (FSUB-02)

1:83                          *           SAME AS FLOATING ADD EXCEPT THAT SIGN CONTROL

1:84                          *           PROCEDURES FOR MQ ARE REVERSED.

1:85                          *

1:86                          *   FLOATING MULTIPLY (FMUL-03)

1:87                          *       EXAMPLE 1. NO EXPONENT MODIFY REQUIRED

1:88                          *       EXAMPLE 2. EXPONENT MODIFY REQUIRED

1:89                          *       EXAMPLE 3. DIGIT FORCE -- MACHINE INFINITY

1:90                          *       EXAMPLE 4. EXPONENT OVERFLOW -- MODIFY PE TO 99

1:91                          *       EXAMPLE 5. SPECIAL CASE 999 X 199

1:92                          *

1:93                          *   FLOATING DIVIDE (FDIV-09)

1:94                          *

1:95

1:96                          * ==========================================================

1:97

1:98    00402 41 00000 00000  FPMAIN NOP   00000, 00000

1:99

1:100   00414 34 00000 00102        RCTY

1:101   00426 34 00000 00102        RCTY

1:102   00438 39 10319 00100        WATY  TITLE

1:103   00450 34 00000 00102        RCTY

1:104                         * ==========================================================

1:105                         *     RUN ALL TESTS

1:106                         *

1:107                         *

1:108                         *ALL TESTS

1:109                         * ==========================================================

1:110

1:111   00462 41 00000 00000  FIRST NOP

1:112

_

1:113   00474 17 08614 00486        BTM   SETS1, *+12,,     SET SW SUBROUTINE

1:114

1:115

_

1:116   00486 17 01236 00600        BTM   TFTST, *+114,,    TFL TEST

_

1:117   00499 06                    DC    2, 06,,,          TFL INSTRUCTION

_

1:118   00501 333333..33330@        DAC   50,======= TRANSMIT FLOATING (TFL 06) TESTS ========@

1:119

1:120

_

1:121   00600 17 01236 00714        BTM   TFTST, *+114,,    FSL TEST

_

1:122   00613 05                    DC    2, 05,,,          FSL INSTRUCTION

_

1:123   00615 333333..33330@        DAC   50,====== FLOATING SHIFT LEFT (FSL 05) TESTS =======@

1:124

1:125

_

1:126   00714 17 01236 00828        BTM   TFTST, *+114,,    FSR TEST

_

1:127   00727 08                    DC    2, 08,,,          FSR INSTRUCTION

_

1:128   00729 333333..33330@        DAC   50,====== FLOATING SHIFT RIGHT (FSR 08) TESTS ======@

1:129

1:130

_

1:131   00828 17 01236 00942        BTM   TFTST, *+114,,    BTFL TEST

_

1:132   00841 07                    DC    2, 07,,,          BTFL INSTRUCTION

_

1:133   00843 333333..33330@        DAC   50,=== BRANCH AND TRANSMIT FLOAT (BTFL 07) TESTS ===@

1:134

1:135

1:136

_

1:137   00942 17 02466 01038        BTM   FATST, *+96,,     FADD TEST

_

1:138   00955 01                    DC    2, 01,,,          FADD INSTRUCTION

_

1:139   00957 21                    DC    2, 21,,,          ADD INSTRUCTION

_

1:140   00959 333333..33330@        DAC   40,====== FLOATING ADD (FADD) TESTS ======@

1:141

1:142

1:143

_

1:144   01038 17 02466 01134        BTM   FATST, *+96,,     FSUB TEST

_

1:145   01051 02                    DC    2, 02,,,          FSUB INSTRUCTION

_

1:146   01053 22                    DC    2, 22,,,          SUBTRACT INSTRUCTION

_

1:147   01055 333333..33330@        DAC   40,====== FLOATING SUB (FSUB) TESTS ======@

1:148

1:149

_

1:150   01134 17 06372 01146        BTM   FMTST, *+12,,     FMUL TEST

1:151

1:152                         * ------ PRINT END OF TESTING MESSAGE AND HALT

1:153

1:154   01146 41 00000 00000  ALDON NOP

1:155   01158 34 00000 00102        RCTY

1:156   01170 34 00000 00102        RCTY

1:157   01182 39 10389 00100        WATY  DONE

1:158   01194 34 00000 00102        RCTY

1:159   01206 48 00000 00000        H     00000, 00000,,    END OF PROGRAM

1:160   01218 48 00000 00000        H     00000, 00000,,    WILL SHOW (H) AS NEXT INSTRUCTION

1:161

1:162

1:163                         * ==========================================================

1:164                         *     TEST SUBROUTINES FOLLOW

1:165                         * ==========================================================

1:166                         * ==========================================================

1:167                         * ==========================================================

1:168

------ 2: fp20-tftest.sps --------------------------------------------------

2:1                           * ========================================================

2:2                           *     TFL, BTFL, FSL, FSR TESTS

2:3                           *

2:4                           *TFTST

2:5                           * ========================================================

2:6     01234    00005              DS    5,,,              RETURN ADDRESS FROM BTM

2:7     01236 41 00000 00000  TFTST NOP   00000, 00000

2:8

2:9                           *  INSTRUCTION  ARG

2:10    01248 26 01283 01235        TF    *+35, TFTST-1,,

_

2:11    01260 12 01283 00101        SM    *+23, 101,,

2:12    01272 26 01290 00000        TF    TFINS+6, 0,,      TO FLOATING POINT INSTRUCTION

2:13    01284 41 00000 00000  TFINS NOP   00000,00000,,     KEEP INSTRUCTION ARG IN P ADDRESS

2:14

2:15                          * TEST TITLE ARG

2:16    01296 26 01331 01235        TF    *+35, TFTST-1,,

_

2:17    01308 12 01331 00099        SM    *+23, 99,,

_

2:18    01320 16 01362 00000        TFM   TFWT+6, 0,,       TO WRITE TITLE

2:19

2:20                          * TITLE

2:21

2:22    01332 34 00000 00102        RCTY

2:23    01344 34 00000 00102        RCTY

_

2:24    01356 39 00000 00100  TFWT  WATY  00000,,6,          MAIN  TEST TITLE

2:25    01368 34 00000 00102        RCTY

2:26

2:27

2:28                          * INITIALIZE TABLE POINTER

2:29

_

2:30    01380 16 10767 00000        TFM   TFINC, 0,,  TABLE POINTER

2:31

2:32                          * POINT TO  TEST DATA TABLES

2:33                          * RESET

_

2:34    01392 16 10782 10802        TFM   TFTBL, TFTBI

_

2:35    01404 16 10787 10837        TFM   TFTBL+5, TFTBD

2:36    01416 26 10777 10782        TF    TFTBP, TFTBL

2:37    01428 26 10782 10787        TF    TFTBP+5, TFTBL+5

2:38

2:39

2:40    01440 41 00000 00000  TFNXT NOP

2:41

2:42                          * ADVANCE TO NEXT ADDRESS IN TABLES

2:43

2:44    01452 21 10777 10767        A     TFTBP,    TFINC,,       ID MESSAGE TABLE

_

2:45    01464 14 10777 00000        CM    TFTBP, 00000, 6,        RETURN ON ZERO ADDRESS

_

2:46    01476 46 01235 01200        BE    TFTST-1,,6,             RETURN

2:47

2:48    01488 21 10782 10767        A     TFTBP+5,  TFINC,,       DATA TABLE

2:49

_

2:50    01500 16 10767 00005        TFM   TFINC, 5

2:51

_

2:52    01512 26 10792 10777        TF    TFI, TFTBP,11

_

2:53    01524 26 10797 10782        TF    TFD, TFTBP+5,11

2:54

2:55                          * START OF CURRENT TEST

2:56    01536 34 00000 00102  TFRPT RCTY

_

2:57    01548 39 10792 00100        WATY  TFI,,6,                 TEST TITLE

2:58    01560 34 00000 00101        SPTY

2:59

2:60                          * FP INSTRUCTION TEST FOLLOW.

2:61                          * TEST DATA FIELD IN ADDRESS REFERENCED

2:62                          * BY TFD ASSUMED TO BE A 100 DIGIT MANTISSA

2:63

2:64                          * SELECT INSTRUCTION (TFINS)

2:65    01572 25 14955 14953        TD    STAT, FALSE

2:66    01584 26 10658 10554        TF    WK1, WK1Z,,             ZERO WK1

2:67    01596 33 10558 00000        CF    WK1-100

2:68    01608 26 10761 10554        TF    WK2, WK1Z,,             ZERO WK2

2:69    01620 33 10661 00000        CF    WK2-100

2:70

2:71    01632 26 02195 10797        TF    TEXCK+11, TFD,,         EXP COMPARE

2:72

2:73                          * WHEN TFL(06)

_

2:74    01644 14 01290 00006        CM    TFINS+6, 06, 10,,   TFL

2:75    01656 47 01740 01200        BNE   TFSL

_

2:76    01668 06 10658 10797        TFL   WK1, TFD,11

2:77    01680 26 01715 10797        TF    *+35, TFD,,             MANTISSA COMPARE

_

2:78    01692 12 01715 00002        SM    *+23, 2

2:79    01704 24 10656 00000        C     WK1-2, 00000,,          MANTISSA

2:80    01716 46 02184 01200        BE    TEXCK,,,                EXPONENT

2:81    01728 49 02220 00000        B     TER

2:82

2:83                          * WHEN FSL(05)

2:84                          *   FSL LEAVES SIGN FLAG IN ORIGINAL POSITION

2:85                          *   THE COMPARE (C) COMPARES THE SHIFTED DATA

2:86                          *   TO THE ORIGINAL. IF THE ORIGINAL IS NEGATIVE

2:87                          *   THEN TO MAKE THE COMPARE WORK THE SHIFTED DATA

2:88                          *   NEEDS A SIGN (FLAG LOW ORDER)

_

2:89    01740 14 01290 00005  TFSL  CM    TFINS+6, 05, 10,,   FSL

2:90    01752 47 01896 01200        BNE   TFSR

2:91    01764 26 01799 10797        TF    *+35, TFD

_

2:92    01776 12 01799 00101        SM    *+23, 101

2:93    01788 31 10557 00000        TR    WK1-101, 00000,,        COPY FP NUMBER TO WORK

2:94    01800 05 10578 10656        FSL   WK1-80, WK1-2

2:95    01812 26 01871 10797        TF    TFSLC+11, TFD

_

2:96    01824 12 01871 00002        SM    TFSLC+11, 2

_

2:97    01836 44 01860 01871        BNF   TFSLC, TFSLC+11, 11

2:98    01848 32 10627 00000        SF    WK1-31

2:99    01860 24 10627 00000  TFSLC C     WK1-31, 00000,,         MANTISSA

2:100   01872 46 02184 01200        BE    TEXCK,,,                EXPONENT

2:101   01884 49 02220 00000        B     TER

2:102

2:103                         * WHEN FSR(08)

2:104                         *  FIRST MAKE LEFT SHIFTED COPY TO WK1 AND CORRECT SIGN

_

2:105   01896 14 01290 00008  TFSR  CM    TFINS+6, 08, 10,,   FSR

2:106   01908 47 02112 01200        BNE   TBTFL

2:107   01920 26 01955 10797        TF    *+35, TFD

_

2:108   01932 12 01955 00002        SM    *+23, 2

2:109   01944 26 10608 00000        TF    WK1-50, 00000,,        COPY FP NUMBER TO WORK

2:110   01956 26 01979 10797        TF    *+23, TFD

2:111   01968 26 10658 00000        TF    WK1, 00000,,             AND EXPONENT

2:112   01980 26 02015 10797        TF    *+35, TFD

_

2:113   01992 12 02015 00002        SM    *+23, 2

2:114   02004 44 02040 00000        BNF   TFSRF, 00000

2:115   02016 33 10608 00000        CF    WK1-50

2:116   02028 32 10656 00000        SF    WK1-2

2:117   02040 08 10656 10608  TFSRF FSR   WK1-2, WK1-50

2:118   02052 26 02087 10797        TF    TFSRC+11, TFD

_

2:119   02064 12 02087 00002        SM    TFSRC+11, 2

2:120   02076 24 10656 00000  TFSRC C     WK1-2, 00000,,          MANTISSA

2:121   02088 46 02184 01200        BE    TEXCK,,,                EXPONENT

2:122   02100 49 02220 00000        B     TER

2:123

2:124

2:125                         * WHEN BTFL(07)

2:126   02112 26 02183 10797  TBTFL TF    *+71, TFD

2:127   02124 26 02159 10797        TF    *+35, TFD

_

2:128   02136 12 02159 00101        SM    *+23, 101

2:129   02148 31 10557 00000        TR    WK1-101, 00000,,        COPY FP NUMBER TO WORK

_

2:130   02160 16 02460 02184        TFM   RETAD, *+24,,           BTFL RETURN

2:131   02172 07 02392 00000        BTFL  BTSUB, 00000,,          USES WK1 AND SETS STAT

2:132

2:133   02184 24 10658 00000  TEXCK C     WK1, 00000,,             EXPONENT

2:134   02196 47 02220 01200        BNE   TER

2:135   02208 25 14955 14952        TD    STAT, TRUE

2:136

2:137

2:138

2:139                         * CHECK COMPARE STATUS

2:140                         * CALL ERROR ROUTINE AND RETURN ITS STATUS IN ESTAT

_

2:141   02220 17 09518 02242  TER   BTM   ERTN, *+22,, TRANSMIT ADDRESS OF NEXT INSTRUCTION

_    _

2:142   02236 1495514954            DSA   STAT, ESTAT

2:143

2:144   02242 43 02266 14954        BD    TFEND,  ESTAT

2:145   02254 49 01536 00000        B     TFRPT,,,    RERUN TEST

2:146

2:147   02266 41 00000 00000  TFEND  NOP   00000, 00000

2:148

2:149   02278 49 01440 00000        B     TFNXT,,,    GOTO NEXT TEST

2:150

2:151

------ 3: fp21-tftest.sps --------------------------------------------------

3:1                           * ---------------------------------------------

3:2                           * SUBROUTINE  STATUS = BTSUB(FP1, WK1)

3:3                           * FP1 IS COPIED TO WK1

3:4                           * BRANCH AND TRANSMIT COPIES WK1 TO SUBROUTINE-BTSUB

3:5                           * BTSUB GETS WK1 CONTENTS JUST BEFORE BTSUB ADDRESS (BTSUB-1)

3:6                           * BTSUB CALLS CF -COMPARE FLOATING SUBROUTINE TO

3:7                           *       COMPARE WK1 TO ORIGINAL FP1

3:8                           * CF RETURNS COMPARE STATUS IN STAT

3:9

3:10                          *

3:11                          *

3:12                          *BTSUB

3:13                          * ---------------------------------------------

3:14

_

3:15    02339 000000..000000        DC    50, 0

3:16    02340 000000..000000        DSC   50, 0

3:17    02390 00                    DSC    2, 0

3:18    02392 41 00000 00000  BTSUB NOP

3:19

3:20

3:21                          *COMPARE MANTISSA AND EXPONENT SEPARATELY

3:22                          * WITH SUBROUTINE CF (COMPARE FLOATING)

_

3:23    02404 17 09922 02432        BTM   CF, *+28,,        TRANSMIT RETURN ADDRESS

_    _   _

3:24    02420 023911..814955        DSA   BTSUB-1, WK1, STAT

3:25

3:26    02432 41 00000 00000        NOP   00000, 00000,,    RETURN TO HERE

3:27

3:28                          * RETURN

3:29                          *     NOTE  BB WILL NOT WORK HERE,

3:30                          *     BECAUSE THIS SUBROUTINE CONTAINS A BTM

_

3:31    02444 49 02460 00000        B     RETAD,,6

3:32

3:33                          * --------------------

3:34                          * SUBROUTINE BTFL RETURN ADDRESS

3:35    02460    00005        RETAD DS    5

3:36

3:37

------ 4: fp30-fadd.sps --------------------------------------------------

4:1                           * ========================================================

4:2                           *     FADD TEST

4:3                           *

4:4                           *FADD

4:5                           * ========================================================

4:6     02465    00005              DS    5,,,              RETURN ADDRESS FROM BTM

4:7     02466 41 00000 00000  FATST NOP   00000, 00000

4:8

4:9                           *  GET ADD / SUBTRACT INST AND TITLE

4:10    02478 26 02513 02465        TF    *+35, FATST-1,,

_

4:11    02490 12 02513 00083        SM    *+23, 83,,

4:12    02502 26 03043 00000        TF    FARIT+1, 0,,      TO FLOATING POINT INSTRUCTION

4:13

4:14    02514 26 02549 02465        TF    *+35, FATST-1,,

_

4:15    02526 12 02549 00081        SM    *+23, 81,,

_

4:16    02538 16 03118 00000        TFM   ADSA, 0,,      FIXED INST TO FIXED ADD/SUB SUB

4:17

4:18    02550 26 02585 02465        TF    *+35, FATST-1,,

_

4:19    02562 12 02585 00079        SM    *+23, 79,,

_

4:20    02574 16 02616 00000        TFM   WTITL+6, 0,,      TO WRITE TITLE

4:21

4:22                          * TITLE

4:23

4:24    02586 34 00000 00102        RCTY

4:25    02598 34 00000 00102        RCTY

_

4:26    02610 39 00000 00100  WTITL WATY  00000,,6,          MAIN FADD / FSUB TEST TITLE

4:27    02622 34 00000 00102        RCTY

4:28

4:29

4:30                          * INITIALIZE TABLE POINTER

4:31

_

4:32    02634 16 11713 00000        TFM   FAINC, 0,,  TABLE POINTER

4:33

_

4:34    02646 14 03043 00002        CM    FARIT+1, 02, 10,  CHOOSE FADD / SUB TABLES

4:35    02658 46 02754 01200        BE    SUBTB

4:36

4:37                          * POINT TO FADD TEST DATA TABLES

4:38                          * RESET

_

4:39    02670 16 11733 11778        TFM   FATBL, FATBI

_

4:40    02682 16 11738 11833        TFM   FATBL+5, FATBP

_

4:41    02694 16 11743 11888        TFM   FATBL+10, FATBQ

4:42    02706 26 11728 11733        TF    ASTBL, FATBL

4:43    02718 26 11733 11738        TF    ASTBL+5, FATBL+5

4:44    02730 26 11738 11743        TF    ASTBL+10, FATBL+10

4:45    02742 49 02826 00000        B     FANXT

4:46

4:47                          * POINT TO FSUB TEST DATA TABLES

4:48                          * RESET

_

4:49    02754 16 11748 11943  SUBTB TFM   FSTBL, FSTBI

_

4:50    02766 16 11753 11998        TFM   FSTBL+5, FSTBP

_

4:51    02778 16 11758 12053        TFM   FSTBL+10, FSTBQ

4:52    02790 26 11728 11748        TF    ASTBL, FSTBL

4:53    02802 26 11733 11753        TF    ASTBL+5, FSTBL+5

4:54    02814 26 11738 11758        TF    ASTBL+10, FSTBL+10

4:55

4:56

4:57    02826 41 00000 00000  FANXT NOP

4:58

4:59                          * ADVANCE TO NEXT ADDRESS IN TABLES

4:60    02838 21 11728 11713        A     ASTBL,    FAINC,,       ID MESSAGE TABLE

_

4:61    02850 14 11728 00000        CM    ASTBL, 00000, 6,        RETURN ON ZERO ADDRESS

_

4:62    02862 46 02465 01200        BE    FATST-1,,6,             RETURN

4:63

4:64    02874 21 11733 11713        A     ASTBL+5,  FAINC,,       P DATA TABLE

4:65    02886 21 11738 11713        A     ASTBL+10, FAINC,,       Q DATA TABLE

4:66

_

4:67    02898 16 11713 00005        TFM   FAINC, 5

4:68

_

4:69    02910 26 11763 11728        TF    FAI, ASTBL,11

_

4:70    02922 26 11768 11733        TF    FAP, ASTBL+5,11

_

4:71    02934 26 11773 11738        TF    FAQ, ASTBL+10,11

4:72

4:73                          * START OF CURRENT FADD TEST

4:74    02946 41 00000 00000  FARPT NOP

4:75    02958 34 00000 00102        RCTY

_

4:76    02970 39 11763 00100        WATY  FAI,,6,          FADD TEST TITLE

4:77    02982 34 00000 00101        SPTY

4:78

4:79                          *--- NEED TO PRESERVE AUGEND - MOVE TO FASUM FIRST

4:80                          *--- AVOID TFL FOR FLOATING POINT TESTS

4:81

_

4:82    02994 26 12205 11768        TF    FASUM, FAP, 11, P POINTS TO EXPONENT

_

4:83    03006 12 11768 00002        SM    FAP, 00002,,    P NOW POINTS TO MANTISSA

_

4:84    03018 26 12203 11768        TF    FASUM-2, FAP, 11,

_

4:85    03030 11 11768 00002        AM    FAP, 00002,,  P AGAIN POINTS TO EXPONENT

4:86

4:87                          * FADD / FSUB INST ARE PASSED IN ARGS, WHICH REPLACE FADD BELOW

4:88

_

4:89    03042 01 12205 11773  FARIT FADD  FASUM, FAQ, 11,   FLOAT ARIITHMETIC

4:90

4:91

4:92                          * INDICATOR CHECK

4:93    03054 47 03078 01400        BNV   *+24

4:94    03066 39 15015 00100        WATY  OVM1,,,           OVERFLOW MESSAGE

4:95    03078 47 03102 01500        BNXV  *+24

4:96    03090 39 15037 00100        WATY  XCKM1,,,          EXPONENT OVERFLOW MESSAGE

4:97

4:98

4:99                          * CALL SUBROUTINE A21

4:100                         * A (21) MANTASSAS AND COMPUTE EXPONENTS

_

4:101   03102 17 03216 03140        BTM   A21,  *+38,,      TRANSMIT RETURN ADDRESS

_    _   _

4:102   03118 000001..514955  ADSA  DSA   00000, -FAP, -FAQ, FASUM, STAT

4:103

4:104

4:105                         * CHECK COMPARE STATUS

4:106                         * CALL ERROR ROUTINE AND RETURN ITS STATUS IN ESTAT

_

4:107   03140 17 09518 03162        BTM   ERTN, *+22,, TRANSMIT ADDRESS OF NEXT INSTRUCTION

_    _

4:108   03156 1495514954            DSA   STAT, ESTAT

4:109

4:110   03162 43 03186 14954        BD    FAEND,  ESTAT

4:111   03174 49 02946 00000        B     FARPT,,,    RERUN TEST

4:112

4:113   03186 41 00000 00000  FAEND  NOP   00000, 00000

4:114

4:115   03198 49 02826 00000        B     FANXT,,,    GOTO NEXT FADD TEST

4:116

------ 5: fp31-fixadd.sps --------------------------------------------------

5:1                           * ---------------------------------------------

5:2                           * SUBROUTINE STATUS = A21(FXARI, FP1, FP2, FP-SUM)

5:3                           *    COMPARES FADD SUM WITH A(21)/EX (EX ADJUST)

5:4                           *    -- OR -- FSUB WITH S(22) PER FIRST ARGUMENT (FXARI)

5:5                           *

5:6                           * REF (1) 227-5630-1 IBM 1620 FLOATING POINT FEATURE CE MANUAL 1962

5:7                           *

5:8                           * VERIFIES FADD RESULT BY USING A(21) TO ADD MANTISSAS

5:9                           * AND SEPARATELY HANDLES EXPONENTS.

5:10                          * FLOATING POINT NUMBER IS CREATED FROM A(21) SUM AND

5:11                          * (ADJUSTED) EXPONENTS

5:12                          *

5:13                          * HANDLES CASES DESCRIBED IN REF (1) FLOATING ADD EXAMPLES

5:14                          *

5:15                          * EXAMPLE 1 PE = QE / D = 0 NO NORMALIZING - NO CARRY OUT

5:16                          * EXAMPLE 2 PE GT QE - D LT L QF SHIFTED RT TO ALIGN DECIMAL POINTS

5:17                          * EXAMPLE 3 PE GT QE - D GT L - Q DISCARDED-RESULT IS P

5:18                          * EXAMPLE 4 PE LT QE- D LT L PF SHIFTED RT TO ALIGN DECIMAL POINTS

5:19                          * EXAMPLE 5 PE LT QE - D GT L P DISCARDED-RESULT IS Q

5:20                          *

5:21                          * EXAMPLE 6 NO NORMALIZING REQUIRED-NO CARRY OUT-SAME AS EXAMPLE 1

5:22                          *

5:23                          * EXAMPLE 7 NORMALIZING SHIFT RT - CARRY OUT- EXP ADJUST

5:24                          * EXAMPLE 8 NORM SHIFT RT-CARRY OT CAUSES EXP OV MACH INFINITY (9...9 99)

5:25                          * EXAMPLE 9 ZERO FRACTION RESULT - MACHINE ZERO (0...0 -99)

5:26                          * EXAMPLE 10 NORMALIZING SHIFT LEFT - SHIFT LEFT- EXP ADJ

5:27                          * EXAMPLE 11 NORMALIZING SHIFT LEFT- EXP OV MACHINE ZERO (0...0 -99)

5:28                          *

5:29                          * USAGE

5:30                          *      BTM   A21,  *+38,,        TRANSMIT RETURN ADDRESS

5:31                          *      DSA   FXARI, -FAP, -FAQ, FASUM, STAT

5:32                          *      NOP   00000, 00000,,    RETURN TO HERE

5:33                          *

5:34                          * FXARI = A OR S ARITHMETIC INSTRUCTION

5:35                          * A21A1 = -FAP (INDIRECT ADDRESS) FLOATING POINT NUMBER

5:36                          * A21A2 = -FAQ (INDIRECT ADDRESS) FLOATING POINT NUMBER

5:37                          * A21D1 = FASUM (DATA FIELD) FLOATING POINT FADD SUM

5:38                          * A21RC = STAT ( DATA, RETURN CODE )

5:39                          * RETURNS STATUS OF COMPARE

5:40                          *

5:41                          *

5:42                          *A21

5:43                          * ---------------------------------------------

5:44    03214    00005              DS    5,,                     RETURN ADDRESS

5:45    03216 41 00000 00000  A21   NOP   00000, 00000

5:46

5:47

5:48                          * GET FIRST ARGUMENT

5:49    03228 26 03263 03215        TF    *+35, A21-1,,

_

5:50    03240 12 03263 00022        SM    *+23, 00022,,

5:51    03252 26 03275 00000        TF    *+23, 0,,

5:52    03264 26 04165 00000        TF    FXAR1+1,  0,,  -- A / S FIXED POINT ARITH INSTRUCTION

5:53    03276 26 03299 03275        TF    *+23, *-1

5:54    03288 26 04789 00000        TF    FXAR2+1,  0,,  -- A / S FIXED POINT ARITH INSTRUCTION

5:55

5:56

5:57                          * GET SECOND ARGUMENT (INDIRECT)

5:58    03300 26 03335 03215        TF    *+35, A21-1,,

_

5:59    03312 12 03335 00017        SM    *+23, 00017,,

5:60    03324 26 03359 00000        TF    *+35, 0,,

5:61    03336 33 03359 00000        CF    *+23

5:62    03348 26 06253 00000        TF    A21A1,  0,,

5:63

5:64

5:65                          * GET THIRD ARGUMENT (INDIRECT)

5:66    03360 26 03395 03215        TF    *+35, A21-1,,

_

5:67    03372 12 03395 00012        SM    *+23, 00012,,

5:68    03384 26 03419 00000        TF    *+35, 0,,

5:69    03396 33 03419 00000        CF    *+23

5:70    03408 26 06258 00000        TF    A21A2,  0,,

5:71

5:72

5:73                          * GET FOURTH ARGUMENT

5:74    03420 26 03455 03215        TF    *+35, A21-1,,

_

5:75    03432 12 03455 00007        SM    *+23, 00007,,

5:76    03444 26 03467 00000        TF    *+23, 0,,

5:77    03456 26 06360 00000        TF    A21D1,  0,,  -- GETS EXP

5:78

5:79    03468 26 03503 03467        TF    *+35, *-1,,

_

5:80    03480 12 03503 00002        SM    *+23, 00002,

5:81    03492 26 06358 00000        TF    A21D1-2,  0,,  -- GETS MANTISSA

5:82

5:83

5:84                          * EXPONENT

5:85                          * HANDLE RESULTANT EXPONENT BASED ON L AND D

5:86                          * AML = MANTISSA LENGTH   AED = ABS(EP-EQ)

5:87

5:88    03504 26 05403 06042        TF    AEP, AZ5,,              CLEAR

5:89    03516 26 05511 06042        TF    AEQ, AZ5,,              CLEAR

_

5:90    03528 21 05403 06253        A     AEP, A21A1, 11,         EXPONENT P

_

5:91    03540 21 05511 06258        A     AEQ, A21A2, 11,         EXPONENT Q

5:92

5:93

5:94                          * SUBTRACT MP AND MQ EXPONENTS   AED = ABS(EP-EQ)

5:95    03552 26 05281 06042        TF    AED, AZ5,,              CLEAR

5:96    03564 21 05281 05403        A     AED, AEP,,              EXPONENT P

5:97    03576 22 05281 05511        S     AED, AEQ,,              EXPONENT Q

5:98    03588 33 05281 00000        CF    AED,,,                  CLEAR SIGN FLAG TO GET ABS VAL

5:99

5:100

5:101                         * MANTISSA

_

5:102   03600 12 06253 00002        SM    A21A1, 00002

_

5:103   03612 12 06258 00002        SM    A21A2, 00002

5:104   03624 26 05397 06243        TF    AMP, AZ101

5:105   03636 26 05505 06243        TF    AMQ, AZ101

_

5:106   03648 26 05397 06253        TF    AMP  , A21A1, 11,       MANTISSA P

_

5:107   03660 26 05505 06258        TF    AMQ  , A21A2, 11,       MANTISSA Q

5:108

5:109

5:110                         * MANTISSA LENGTH OF FIRST ARG IN AML

5:111                         *     L REPRESENTS MANTISSA LENGTH

_

5:112   03672 16 05276 00001        TFM   AML, 00001,

5:113   03684 26 05296 06253        TF    AMPA, A21A1,,           AMPA HIGH ORDER ADDRESS OF AMP

5:114

_

5:115   03696 12 05296 00001  AL    SM    AMPA, 1

_

5:116   03708 11 05276 00001        AM    AML, 1

_

5:117   03720 44 03696 05296        BNF   AL, AMPA, 11

5:118

5:119

5:120                         * ALIGN DECIMAL POINTS

5:121   03732 24 05403 05511        C     AEP, AEQ

5:122   03744 46 04104 01200        BE    ADDM,,,                 NO ALIGN-ADD MANTISSAS UNCHANGED

5:123   03756 47 03780 01300        BL    ADJP,,,                 PE LT QE

5:124   03768 46 03936 01100        BH    ADJQ,,,                 PE GT QE

5:125

5:126

5:127                         * ADJUST P MANTISSA

5:128                         *   PE LT QE - D GT L - P DISCARDED-RESULT IS Q

5:129   03780 24 05281 05276  ADJP  C     AED, AML,,              COMPARE D TO L

5:130   03792 47 03840 01300        BL    ADJP1

5:131   03804 26 05613 05505        TF    AMPQ, AMQ

5:132   03816 26 05619 05511        TF    AEPQ, AEQ

5:133   03828 49 05148 00000        B     AMKFP,,,          GOTO MAKE FP NUMBER FROM Q ONLY

5:134

_

5:135   03840 16 03875 05397  ADJP1 TFM   *+35, AMP,,             RESTORE ADDRESS IN TF BELOW

5:136   03852 22 03875 05281        S     *+23, AED,,             ALIGN DECIMAL POINTS

5:137   03864 26 05397 05397        TF    AMP, AMP

_

5:138   03876 16 03918 05397        TFM   *+42, AMP,,             RESTORE ADDRESS IN MF BELOW

5:139   03888 22 03918 05276        S     *+30, AML

_

5:140   03900 11 03918 00001        AM    *+18, 00001

5:141   03912 71 05397 05397        MF    AMP, AMP

5:142   03924 49 04104 00000        B     ADDM

5:143

5:144

5:145                         * ADJUST Q MANTISSA

5:146                         * PE GT QE - D GT L - Q DISCARDED-RESULT IS P

5:147   03936 41 00000 00000  ADJQ  NOP   00000, 00000,,

5:148   03948 24 05281 05276        C     AED, AML,,              COMPARE D TO L

5:149   03960 47 04008 01300        BL    ADJQ1

5:150   03972 26 05613 05397        TF    AMPQ, AMP

5:151   03984 26 05619 05403        TF    AEPQ, AEP

5:152   03996 49 05148 00000        B     AMKFP,,,          GOTO MAKE FP NUMBER FROM P ONLY

5:153

_

5:154   04008 16 04043 05505  ADJQ1 TFM   *+35, AMQ,,             RESTORE ADDRESS IN TF BELOW

5:155   04020 22 04043 05281        S     *+23, AED,,             ALIGN DECIMAL POINTS

5:156   04032 26 05505 05505        TF    AMQ, AMQ

_

5:157   04044 16 04086 05505        TFM   *+42, AMQ,,             RESTORE ADDRESS IN MF BELOW

5:158   04056 22 04086 05276        S     *+30, AML

_

5:159   04068 11 04086 00001        AM    *+18, 00001

5:160   04080 71 05505 05505        MF    AMQ, AMQ

5:161   04092 49 04104 00000        B     ADDM

5:162

5:163

5:164                         * ADD (ADJUSTED) P TO Q MANTISSAS

5:165   04104 41 00000 00000  ADDM  NOP   00000, 00000,,

5:166   04116 25 05622 14953        TD    ANRM, FALSE,,           CLEAR NORMALIZE FLAG

5:167   04128 25 05621 14953        TD    AOV, FALSE,,            CLEAR OVERFLOW FLAG

5:168   04140 26 05613 06243        TF    AMPQ, AZ101,,           CLEAR

5:169   04152 26 05613 05397        TF    AMPQ, AMP

5:170   04164 21 05613 05505  FXAR1 A     AMPQ, AMQ,,             ARGS MAY REPLACE A WITH S

5:171

5:172

5:173                         * CARRY-OUT OVERFLOW INDICATOR CHECK

5:174   04176 47 04212 01400        BNV   ASHFT

5:175   04188 25 05621 14952        TD    AOV, TRUE,,             SET OVERFLOW FLAG

5:176   04200 49 04548 00000        B     AZCK

5:177

5:178

5:179                         * NOMRALIZE MANTISSA IN AMPQ

5:180                         *   SHIFT LEFT

5:181                         *   COUNT LEADING ZEROS IN AZCNT

5:182                         *   REQUIRES EXPONENT ADJUST

5:183   04212 41 00000 00000  ASHFT NOP   00000, 00000

5:184   04224 24 05613 06243        C     AMPQ, AZ101,,           ZERO MANTISSA CHECK

5:185   04236 46 04572 01200        BE    AZRO

5:186

5:187   04248 25 05622 14953        TD    ANRM, FALSE,,           CLEAR NORMALIZE FLAG

_

5:188   04260 16 05291 00000        TFM   AZCNT, 00000,,          CLEAR LEADING ZERO COUNTER

5:189

_

5:190   04272 16 04319 05613        TFM   *+47, AMPQ

5:191   04284 22 04319 05276        S     *+35, AML

_

5:192   04296 11 04319 00001        AM    *+23, 00001

5:193   04308 43 04548 05613        BD    AZCK, AMPQ,,            LEADING ZERO CHECK

5:194

5:195   04320 26 06034 06243        TF    AWK1, AZ101,,           CLEAR WORK

5:196   04332 25 05622 14952        TD    ANRM, TRUE,,            SET NORMALIZE FLAG

_

5:197   04344 16 05286 00000        TFM   AFLG, 00000

_

5:198   04356 16 05296 05613        TFM   AMPA, AMPQ,,            POINTS HI MANTISSA ADDR

5:199

_

5:200   04368 12 05296 00001  AFLAG SM    AMPA, 00001,,           SEARCH FOR ORIGINAL FLAG

_

5:201   04380 44 04368 05296        BNF   AFLAG, AMPA, 11

_

5:202   04392 33 05296 00000        CF    AMPA, 00000, 6,         CLEAR IT

5:203

_

5:204   04404 43 04464 05296  ADIGT BD    ASF, AMPA, 11,          1ST DIGIT TO SHIFT LEFT

_

5:205   04416 11 05291 00001        AM    AZCNT, 00001,,          COUNT LEADING ZEROS

_

5:206   04428 11 05286 00001        AM    AFLG, 00001

_

5:207   04440 11 05296 00001        AM    AMPA, 00001

5:208   04452 49 04404 00000        B     ADIGT

5:209

_

5:210   04464 16 04530 05613  ASF   TFM   ASF2+6, AMPQ,,          INITIALIZE TF P ADDRESS

_

5:211   04476 32 05296 00000        SF    AMPA, 00000, 6,         FLAG 1ST DIGIT

5:212   04488 26 06034 05613        TF    AWK1, AMPQ,,            NOW LEFT SHIFT MANTISSA

5:213   04500 26 05613 06243        TF    AMPQ, AZ101

5:214   04512 22 04530 05286        S     *+18, AFLG

5:215   04524 26 05613 06034  ASF2  TF    AMPQ, AWK1,,            SHIFT COMPLETE

5:216   04536 49 04824 00000        B     EXADJ,,,                GOTO EXPONENT ADJUST

5:217

5:218

5:219

5:220                         * ZERO CHECK GOTO MACHINE ZERO

5:221   04548 24 05613 06243  AZCK  C     AMPQ, AZ101,,           ZERO MANTISSA CHECK

5:222   04560 47 04596 01200        BNE   ACO

_   _

5:223   04572 16 05619 00099  AZRO  TFM   AEPQ, -99,,             MAKE MACHINE ZERO

5:224   04584 49 05148 00000        B     AMKFP,,,                GOTO MAKE FP

5:225

5:226

5:227                         * NORMALIZE SHIFT RIGHT

5:228                         *  - CARRY OUT- EXP ADJUST

_

5:229   04596 16 04710 05397  ACO   TFM   ACMF1+6, AMP,,          INITIALIZE MF INSTRUCTIONS

_

5:230   04608 16 04715 05397        TFM   ACMF1+11, AMP

_

5:231   04620 16 04758 05397        TFM   ACMF2+6, AMP,,

_

5:232   04632 16 04763 05397        TFM   ACMF2+11, AMP

5:233   04644 43 04668 05621        BD    *+24, AOV

5:234   04656 49 04824 00000        B     EXADJ,,,                NO CARRY OUT GOTO EXP ADJ

5:235   04668 22 04715 05276        S     *+47, AML

_

5:236   04680 11 04715 00001        AM    *+35, 00001

5:237   04692 22 04710 05276        S     *+18, AML

5:238   04704 71 05397 05397  ACMF1 MF    AMP, AMP

5:239   04716 22 04763 05276        S     *+47, AML

_

5:240   04728 11 04763 00001        AM    *+35, 00001

5:241   04740 22 04758 05276        S     *+18, AML

5:242   04752 71 05505 05505  ACMF2 MF    AMQ, AMQ

5:243   04764 26 05613 06243        TF    AMPQ, AZ101,,           CLEAR

5:244   04776 26 05613 05397        TF    AMPQ, AMP

5:245   04788 21 05613 05505  FXAR2 A     AMPQ, AMQ,,             ARGS MAY REPLACE A WITH S

5:246   04800 71 05612 05613        MF    AMPQ-1, AMPQ

5:247   04812 26 05613 05612        TF    AMPQ, AMPQ-1

5:248

5:249

5:250                         * ADJUST EXPONENT

5:251                         *     ARITHMETIC OVERFLOW

5:252                         *     MACHINE INFINITY

5:253                         *     MANTISSA LEFT SHIFT FOR NORMALIZATION

5:254   04824 26 05619 06042  EXADJ TF    AEPQ, AZ5

5:255   04836 26 05619 05403        TF    AEPQ, AEP

5:256   04848 24 05403 05511        C     AEP, AEQ

5:257   04860 46 04884 01300        BNL   *+24

5:258   04872 26 05619 05511        TF    AEPQ, AEQ

5:259   04884 43 05088 05622        BD    AELS, ANRM,,            NORMALIZE LEFT SHIFT CHECK

5:260   04896 43 04920 05621        BD    *+24, AOV,,,            ARITHMETIC OVERFLOW CHECK

5:261   04908 49 05148 00000        B     AMKFP

5:262

_

5:263   04920 14 05619 00099        CM    AEPQ, 00099,,           EXP 99 THEN MACH INF

5:264   04932 46 04968 01300        BNL   AINF

_

5:265   04944 11 05619 00001        AM    AEPQ, 1,,               ARITH OV EXP ADJUST

5:266   04956 49 05148 00000        B     AMKFP

5:267

5:268                         * NORMALIZE SHIFT RIGHT-CARRY OUT

5:269                         *   EXPONENT OVERFLOW MACH INFINITY

_

5:270   04968 16 05070 05613  AINF  TFM   AINF2+6, AMPQ,,         INITIALIZE SF

5:271   04980 44 05028 05613        BNF   AINF1, AMPQ

5:272   04992 26 05613 05929        TF    AMPQ, A9-3,,            MOVE NINES

5:273   05004 32 05613 00000        SF    AMPQ,,,                 REPLACE SIGN FLAG

5:274   05016 49 05040 00000        B     *+24

5:275   05028 26 05613 05929  AINF1 TF    AMPQ, A9-3,,            MOVE NINES

5:276   05040 22 05070 05276        S     *+30, AML

_

5:277   05052 11 05070 00001        AM    *+18, 00001

5:278   05064 32 05613 00000  AINF2 SF    AMPQ

5:279   05076 49 05148 00000        B     AMKFP

5:280

5:281                         * NORMALIZING SHIFT LEFT

5:282                         *    CHECK FOR EXP OVERFLOW

5:283                         *    SET TO MACHINE ZERO

5:284   05088 22 05619 05291  AELS  S     AEPQ, AZCNT,,           ADJ EXP FOR LEFT SHIFT

_   _

5:285   05100 14 05619 00100        CM    AEPQ, -100

5:286   05112 47 05148 01200        BNE   AMKFP

_   _

5:287   05124 16 05619 00099        TFM   AEPQ, -99

5:288   05136 26 05613 06243        TF    AMPQ, AZ101

5:289

5:290

5:291                         * PUT SUM AND ADJUSTED EXPONENT

5:292                         *     INTO FLOATING POINT MM...MM EE

5:293   05148 71 05618 05615  AMKFP MF    AEPQ-1, AEPQ-4

5:294   05160 26 08153 05619        TF    PQFP,  AEPQ

5:295   05172 26 08151 05613        TF    PQFP-2, AMPQ

5:296

5:297

5:298                         * COMPARE FADD SUM TO A21 SUM

5:299                         * A21RC EQ TRUE IF EQUAL OTW FALSE

_

5:300   05184 17 09922 05212        BTM   CF, *+28,,        COMPARE FLOATING SUBROUTINE

_    _   _

5:301   05200 063600..306366        DSA   A21D1, PQFP, A21RC

5:302

5:303

5:304                         * SEND RETURN CODE TO CALLING PROGRAM

5:305   05212 26 05247 03215        TF    *+35, A21-1,,

_

5:306   05224 12 05247 00002        SM    *+23, 2,,

5:307   05236 26 05254 00000        TF    *+18, 0,,

5:308   05248 25 00000 06366        TD    0, A21RC,,

5:309

5:310

5:311                         * RETURN

_

5:312   05260 49 03215 00000  FACR  B     A21-1,,6,                RETURN TO CALLING PROGRAM

5:313

5:314

5:315

5:316

5:317

5:318                         * ---- -- WORK AREAS FOR A (21) INSTRUCTION

_

5:319   05276 00000           AML   DC     5, 0,,,          MANTISSA LENGTH

_

5:320   05281 00000           AED   DC     5, 0,,,          EXP DIFFERENCE D = ABS(EP-EQ)

5:321

_

5:322   05286 00000           AFLG  DC     5, 0,,,          MANTISSA FLAG ADDRESS

_

5:323   05291 00000           AZCNT DC     5, 0,,,          MANTISSA LEADING ZEROS COUNT

5:324

_

5:325   05296 00000           AMPA  DC     5,0,,            POINTER TO AMP

5:326

_

5:327   05346 000000..000000        DC    50, 0,,,          MP MANTISSA FIELD

5:328   05347 000000..000000        DSC   50, 0,,,          SIZE IS 101 FOR CARRY/OVERFLOW

5:329   05397 0               AMP   DSC    1, 0

5:330   05398 @                     DC     1, @

5:331

_

5:332   05403 00000           AEP   DC     5,0,,,            P EXPONENT

5:333   05404 @                     DC     1, @

5:334

_

5:335   05454 000000..000000        DC    50, 0,,,          MQ MANTISSA FIELD

5:336   05455 000000..000000        DSC   50, 0,,,          SIZE IS 101 FOR CARRY/OVERFLOW

5:337   05505 0               AMQ   DSC    1, 0

5:338   05506 @                     DC     1, @

5:339

_

5:340   05511 00000           AEQ   DC     5,0,,,            Q EXPONENT

5:341   05512 @                     DC     1, @

5:342

_

5:343   05562 000000..000000        DC    50, 0,,,          MP  +  MQ SUM MANTISSA FIELD

5:344   05563 000000..000000        DSC   50, 0,,,

5:345   05613 0               AMPQ  DSC    1, 0

5:346   05614 @                     DC     1, @

5:347

_

5:348   05619 00000           AEPQ  DC     5,0,,,            FINAL EXPONENT

5:349   05620 @                     DC     1, @

5:350

_

5:351   05621 0               AOV   DC     1, 0,,,          OVERFLOW FLAG FOR CARRY OUT

_

5:352   05622 0               ANRM  DC     1, 0,,,          NORMALIZE FLAG

5:353

5:354                         *              123456789 123456789 123456789 123456789 1234567890

_

5:355   05625 417271..47450@  AOVM  DAC   21,A21 OVERFLOW MESSAGE@

_

5:356   05667 417271..45000@  AEOVM DAC   31,A21 EXPONENT OVERFLOW MESSAGE@

_

5:357   05777 000000..000000        DC    50, 0,,,          MANTISSA + EXP

5:358   05778 000000..000000        DSC   50, 0

_

5:359   05829 00              APQFP DC     2, 0

5:360   05830 @                     DC     1, @

5:361

5:362

5:363                         *               123456789 123456789 123456789 123456789 1234567890

_

5:364   05880 999999..999999        DC    50, 99999999999999999999999999999999999999999999999999

5:365   05881 999999..999999        DSC   50, 99999999999999999999999999999999999999999999999999

_

5:366   05932 99              A9    DC     2, 99

5:367   05933 @                     DC     1, @

5:368

_

5:369   05983 000000..000000        DC    50, 0

5:370   05984 000000..000000        DSC   50, 0

_

5:371   06034 0               AWK1  DC     1, 0

5:372   06035 @                     DC     1, @

5:373

_

5:374   06037 00              AZ2    DC     2, 0

_

5:375   06042 00000           AZ5    DC     5, 0

5:376

_

5:377   06092 000000..000000         DC    50, 0,,,         100 ZEROS

5:378   06093 000000..000000         DSC   49, 0

5:379   06142 0               AZ100  DSC    1, 0

5:380

_

5:381   06192 000000..000000         DC    50, 0,,,         101 ZEROS

5:382   06193 000000..000000         DSC   50, 0

5:383   06243 0               AZ101  DSC    1, 0

5:384

5:385

_

5:386   06248 00000           AMPAA DC     5, 0,,,          HIGH ORDER ADDRESS OF AMPMA

5:387

_

5:388   06253 00000           A21A1 DC     5, 0,,,          ARG1

_

5:389   06258 00000           A21A2 DC     5, 0,,,          ARG2

5:390

_

5:391   06308 000000..000000        DC    50, 0

5:392   06309 000000..000000        DSC   50, 0

_

5:393   06360 00              A21D1 DC     2, 0 ,,,         FADD CALLING PROGRAM PRODUCT

5:394   06361 @                     DC     1, @

5:395

_

5:396   06366 00000           A21RC DC     5, 0,,,          RETURN CODE

5:397

------ 6: fp40-fmul.sps --------------------------------------------------

6:1                           * ========================================================

6:2                           *     FMUL TEST

6:3                           *

6:4                           *FMUL

6:5                           * ========================================================

6:6

6:7     06371    00005              DS    5,,,             RETURN ADDRESS FROM BTM

6:8     06372 41 00000 00000  FMTST NOP   00000, 00000

6:9

6:10    06384 34 00000 00102        RCTY

6:11    06396 34 00000 00102        RCTY

6:12    06408 39 13551 00100        WATY  FMULT,,,          MAIN FMUL TEST TITLE

6:13    06420 34 00000 00102        RCTY

6:14

6:15

6:16                          * INITIALIZE TABLE POINTER

6:17

_

6:18    06432 16 13644 00000        TFM   FMINC, 0,,  TABLE POINTER

6:19

6:20                          * POINT TO  TEST DATA TABLES

6:21                          * RESET

_

6:22    06444 16 13664 13694        TFM   FMTBL, FMTBI

_

6:23    06456 16 13669 13734        TFM   FMTBL+5, FMTBP

_

6:24    06468 16 13674 13774        TFM   FMTBL+10, FMTBQ

6:25    06480 26 13659 13664        TF    FMTP, FMTBL

6:26    06492 26 13664 13669        TF    FMTP+5, FMTBL+5

6:27    06504 26 13669 13674        TF    FMTP+10, FMTBL+10

6:28

6:29    06516 41 00000 00000  FMNXT NOP

6:30

6:31    06528 21 13659 13644        A     FMTP,    FMINC,,       ID MESSAGE TABLE

_

6:32    06540 14 13659 00000        CM    FMTP, 00000, 6,        RETURN ON ZERO ADDRESS

_

6:33    06552 46 06371 01200        BE    FMTST-1,,6,             RETURN

6:34

6:35    06564 21 13664 13644        A     FMTP+5,  FMINC,,       MULTIPLICAND TABLE

6:36    06576 21 13669 13644        A     FMTP+10, FMINC,,       MULTIPLIERS TABLE

6:37

_

6:38    06588 16 13644 00005        TFM   FMINC, 5

6:39

_

6:40    06600 26 13679 13659        TF    FMI, FMTP,11

_

6:41    06612 26 13684 13664        TF    FMP, FMTP+5,11

_

6:42    06624 26 13689 13669        TF    FMQ, FMTP+10,11

6:43

6:44                          * START OF CURRENT FMUL TEST

6:45    06636 41 00000 00000  FMRPT NOP

6:46    06648 34 00000 00102        RCTY

_

6:47    06660 39 13679 00100        WATY  FMI,,6,          TEST TITLE

6:48    06672 34 00000 00101        SPTY

6:49

6:50                          *--- NEED TO PRESERVE MULTIPLICAND - MOVE TO FMPRD FIRST

6:51                          *--- AVOID TFL FOR FLOATING POINT TESTS

6:52

_

6:53    06684 26 13911 13684        TF    FMPRD, FMP, 11, FMP POINTS TO EXPONENT

_

6:54    06696 12 13684 00002        SM    FMP, 00002,,    FMP NOW POINTS TO MANTISSA

_

6:55    06708 26 13909 13684        TF    FMPRD-2, FMP, 11,

_

6:56    06720 11 13684 00002        AM    FMP, 00002,,    FMA AGAIN POINTS TO EXPONENT

6:57

_

6:58    06732 03 13911 13689        FMUL  FMPRD, FMQ, 11

6:59

6:60                          * INDICATOR CHECK

6:61    06744 47 06768 01400        BNV   *+24

6:62    06756 39 15015 00100        WATY  OVM1,,,           OVERFLOW MESSAGE

6:63    06768 47 06792 01500        BNXV  *+24

6:64    06780 39 15037 00100        WATY  XCKM1,,,          EXPONENT OVERFLOW MESSAGE

6:65

6:66                          * CALL SUBROUTINE M23

6:67                          * M (23) MANTASSAS AND COMPUTE EXPONENTS

_

6:68    06792 17 06900 06824        BTM   M23,  *+32,,      TRANSMIT RETURN ADDRESS

_   __   _

6:69    06808 136841..114955        DSA   -FMP, -FMQ, FMPRD, STAT

6:70

6:71

6:72                          * CHECK COMPARE STATUS

6:73                          * CALL ERROR ROUTINE AND RETURN ITS STATUS IN ESTAT

_

6:74    06824 17 09518 06846        BTM   ERTN, *+22,, TRANSMIT ADDRESS OF NEXT INSTRUCTION

_    _

6:75    06840 1495514954            DSA   STAT, ESTAT

6:76

6:77    06846 43 06870 14954        BD    FMEND,  ESTAT

6:78    06858 49 06636 00000        B     FMRPT,,,    RERUN TEST

6:79

6:80    06870 41 00000 00000  FMEND  NOP   00000, 00000

6:81    06882 49 06516 00000        B     FMNXT,,,          GOTO NEXT FMUL TEST

6:82

------ 7: fp41-fixmul.sps --------------------------------------------------

7:1                           * ---------------------------------------------

7:2                           * SUBROUTINE STATUS = M23(FP1, FP2, FP-PRODUCT)

7:3                           *    COMPARES FMUL PRODUCT WITH M(23)/EX-SUM

7:4                           * REF (1) 227-5630-1 IBM 1620 FLOATING POINT FEATURE CE MANUAL 1962

7:5                           *

7:6                           * VERIFIES FMUL RESULT BY USING M(23) TO MULTIPLY MANTISSAS

7:7                           * AND SEPARATELY ADDS EXPONENTS.

7:8                           * FLOATING POINT NUMBER IS CREATED FROM M(23) PRODUCT AND

7:9                           * (ADJUSTED) SUM OF EXPONENTS

7:10                          *

7:11                          * HANDLES CASES DESCRIBED IN REF (1) FLOATING MULTIPLY EXAMPLES

7:12                          *

7:13                          * FMUL EXAMPLE 1. NO EXPONENT MODIFY REQUIRED

7:14                          * FMUL EXAMPLE 2. EXPONENT MODIFY REQUIRED

7:15                          * FMUL EXAMPLE 3. DIGIT FORCE -- MACHINE INFINITY

7:16                          * FMUL EXAMPLE 4. EXPONENT OVERFLOW -- MODIFY PE TO 99

7:17                          * FMUL EXAMPLE 5. SPECIAL CASE 999 X 199

7:18                          *

7:19                          *

7:20                          * USAGE

7:21                          *      BTM   M23,  *+32,,        TRANSMIT RETURN ADDRESS

7:22                          *      DSA   -FMP, -FMQ, FMPRD, STAT

7:23                          *      NOP   00000, 00000,,    RETURN TO HERE

7:24                          *

7:25                          * M23A1 = -FMP (INDIRECT ADDRESS) FLOATING POINT NUMBER

7:26                          * M23A2 = -FMQ (INDIRECT ADDRESS) FLOATING POINT NUMBER

7:27                          * M23D1 = FMPRD (DATA FIELD) FLOATING POINT FMUL PRODUCT

7:28                          * M23RC = STAT ( DATA, RETURN CODE )

7:29                          * RETURNS STATUS OF COMPARE

7:30                          *

7:31                          *

7:32                          *M23

7:33                          * ---------------------------------------------

7:34    06898    00005              DS    5,,                     RETURN ADDRESS

7:35    06900 41 00000 00000  M23   NOP   00000, 00000

7:36

7:37                          * GET FIRST ARGUMENT (INDIRECT)

7:38    06912 26 06947 06899        TF    *+35, M23-1,,

_

7:39    06924 12 06947 00016        SM    *+23, 00016,,

7:40    06936 26 06971 00000        TF    *+35, 0,,

7:41    06948 33 06971 00000        CF    *+23

7:42    06960 26 08498 00000        TF    M23A1,  0,,

7:43

7:44

7:45                          * GET SECOND ARGUMENT (INDIRECT)

7:46    06972 26 07007 06899        TF    *+35, M23-1,,

_

7:47    06984 12 07007 00011        SM    *+23, 00011,,

7:48    06996 26 07031 00000        TF    *+35, 0,,

7:49    07008 33 07031 00000        CF    *+23

7:50    07020 26 08503 00000        TF    M23A2,  0,,

7:51

7:52

7:53                          * GET THIRD ARGUMENT

7:54    07032 26 07067 06899        TF    *+35, M23-1,,

_

7:55    07044 12 07067 00006        SM    *+23, 00006,,

7:56    07056 26 07079 00000        TF    *+23, 0,,

7:57    07068 26 08605 00000        TF    M23D1,  0,,  -- GETS EXP

7:58

7:59    07080 26 07115 07079        TF    *+35, *-1,,

_

7:60    07092 12 07115 00002        SM    *+23, 00002,

7:61    07104 26 08603 00000        TF    M23D1-2,  0,,  -- GETS MANTISSA

7:62

7:63

7:64                          * ADD MP AND MQ EXPONENTS

7:65    07116 26 08159 08373        TF    SUMEX, MZ5,,            CLEAR

_

7:66    07128 21 08159 08498        A     SUMEX, M23A1, 11,       EXPONENT 1

_

7:67    07140 21 08159 08503        A     SUMEX, M23A2, 11,       EXPONENT 2

7:68

7:69                          * MANTISSA

_

7:70    07152 12 08498 00002        SM    M23A1, 00002

_

7:71    07164 12 08503 00002        SM    M23A2, 00002

_

7:72    07176 26 07848 08498        TF    MPMA, M23A1, 11,        MANTISSA 1

_

7:73    07188 26 07949 08503        TF    MQMA, M23A2, 11,        MANTISSA 2

7:74

7:75                          * MANTISSA LENGTH OF FIRST ARG

7:76                          *     L REPRESENTS MANTISSA LENGTH

_

7:77    07200 16 07748 00001        TFM   FML, 00001

7:78    07212 26 08493 08498        TF    MPMAA, M23A1

7:79

_

7:80    07224 12 08493 00001  ML    SM    MPMAA, 1

_

7:81    07236 11 07748 00001        AM    FML, 1

_

7:82    07248 44 07224 08493        BNF   ML, MPMAA, 11

7:83

7:84                          * IF EXP SUM GT 100 THEN RESULT IS MACHINE INFINITY 99...9 99

7:85                          * IF EXP SUM EQ 100 ONLY SET EXP TO 99

_

7:86    07260 14 08159 00100        CM    SUMEX, 100

7:87    07272 47 07392 01300        BL    MUL,,,                  SUM EXP LT 100

7:88    07284 46 07380 01200        BE    ME100,,,                SUM EXP EQ 100

7:89

7:90    07296 26 08159 08262        TF    SUMEX, M9,,             SUM EXP GT 100 SET SUM EXP TO 99

7:91    07308 26 08363 08260        TF    MWK1-2, M9-2,,          COPY 9S TO WORK AREA

7:92    07320 22 07350 07748        S     *+30, FML

_

7:93    07332 11 07350 00001        AM    *+18, 00001,

7:94    07344 32 08363 00000        SF    MWK1-2

7:95    07356 26 08050 08363        TF    PQMA, MWK1-2,,          9S IN MANTISSA PRODUCT FIELD

7:96    07368 49 07596 00000        B     MKFP,,,                 GOTO MAKE FP NUM FOR COMPARE

7:97

7:98    07380 26 08159 08262  ME100 TF    SUMEX, M9,,           SUM EXP EQ 100 SET SUM EXP TO 99

7:99

7:100                         * MULTIPLY MANTISSAS (P,Q) USING M (23)

_

7:101   07392 26 08478 08424  MUL   TF    PAREA, MZ100,6,          100 ZEROS TO PRODUCT AREA (99)

7:102   07404 23 07848 07949        M     MPMA, MQMA,,            P MANTISSA X Q MANTISSA

7:103

7:104                         * CHECK FOR EXPONENT CASE - PRODUCT LENGTH LT 2L

7:105                         * HIGH ORDER  0

7:106                         *   PROD-HIGH = 100 -2L

_

7:107   07416 16 08483 00100        TFM   PAR1, 00100,,          PRODUCT AREA (99) + 1

7:108   07428 22 08483 07748        S     PAR1, FML,,             100 - L

7:109   07440 22 08483 07748        S     PAR1, FML,,             100 - L

_

7:110   07452 43 07548 08483        BD    PRSH, PAR1,11,          IF HIGH ORDER  0 THEN MOVE FLAG

7:111   07464 26 08488 08483        TF    PARW, PAR1

_

7:112   07476 11 08488 00001        AM    PARW, 00001,

_     _

7:113   07488 71 08488 08483        MF    PARW, PAR1,611

_

7:114   07500 12 07748 00001        SM    FML, 00001,

7:115

_

7:116   07512 14 08159 00099        CM    SUMEX, 00099,,         IF OV KEEP 99

7:117   07524 46 07548 01200        BE    PRSH

_

7:118   07536 12 08159 00001        SM    SUMEX, 00001,,         MODIFY EXP IF HI ORD DIGIT EQ 0

7:119

7:120                         * SHIFT PRODUCT L (MANTISSA LENGTH) AND MOVE SIGN FLAG

7:121   07548 26 08488 08478  PRSH  TF    PARW, PAREA

7:122   07560 22 08488 07748        S     PARW, FML,,

_     _

7:123   07572 71 08488 08478        MF    PARW, PAREA,611

7:124

7:125                         * MOVE PRODUCT TO RESULT AREA

_

7:126   07584 26 08050 08488        TF    PQMA, PARW , 11,

7:127

7:128                         * PUT PRODUCT AND EXPONENT SUM  INTO FLOATING POINT MM...MM EE

7:129   07596 41 00000 00000  MKFP  NOP   0000,0000

7:130   07608 71 08158 08155        MF    SUMEX-1, SUMEX-4

7:131   07620 26 08153 08159        TF    PQFP,  SUMEX

7:132   07632 26 08151 08050        TF    PQFP-2, PQMA

7:133

7:134                         * COMPARE FMUL PRODUCT TO M23 PRODUCT

7:135                         * M23RC EQ TRUE IF EQUAL OTW FALSE

_

7:136   07644 17 09922 07672        BTM   CF, *+28,,        TRANSMIT RETURN ADDRESS

_    _   _

7:137   07660 086050..308607        DSA   M23D1, PQFP, M23RC

7:138

7:139                         * SEND RETURN CODE TO CALLING PROGRAM

7:140   07672 26 07707 06899        TF    *+35, M23-1,,

_

7:141   07684 12 07707 00001        SM    *+23, 1,,

7:142   07696 26 07714 00000        TF    *+18, 0,,

7:143   07708 25 00000 08607        TD    0, M23RC,,

7:144

7:145   07720 41 00000 00000  FMCR  NOP   00000, 00000

7:146

7:147                         * RETURN

_

7:148   07732 49 06899 00000        B     M23-1,,6,                RETURN TO CALLING PROGRAM

7:149

7:150                         * ---- -- WORK AREAS FOR M (23) INSTRUCTION

_

7:151   07748 00000           FML   DC     5, 0,,,          MANTISSA LENGTH

7:152

_

7:153   07798 000000..000000        DC    50, 0,,,          MP MANTISSA FIELD

7:154   07799 000000..000000        DSC   49, 0,,,

7:155   07848 0               MPMA  DSC    1, 0,,,

7:156   07849 @                     DC     1, @

7:157

_

7:158   07899 000000..000000        DC    50, 0,,,          MQ MANTISSA FIELD

7:159   07900 000000..000000        DSC   49, 0,,,

7:160   07949 0               MQMA  DSC    1, 0,,,

7:161   07950 @                     DC     1, @

7:162

_

7:163   08000 000000..000000        DC    50, 0,,,          MP  X  MQ PRODUCT MANTISSA FIELD

7:164   08001 000000..000000        DSC   49, 0,,,

7:165   08050 0               PQMA  DSC    1, 0,,,

7:166   08051 @                     DC     1, @

7:167

_

7:168   08101 000000..000000        DC    50, 0,,,          MANTISSA + EXP

7:169   08102 000000..000000        DSC   50, 0

_

7:170   08153 00              PQFP  DC     2, 0

7:171   08154 @                     DC     1, @

7:172

_

7:173   08159 00000           SUMEX DC     5, 0,,,          EXPONENT SUM FIELD

7:174   08160 @                     DC     1, @

7:175

7:176                         *               123456789 123456789 123456789 123456789 1234567890

_

7:177   08210 999999..999999        DC    50, 99999999999999999999999999999999999999999999999999

7:178   08211 999999..999999        DSC   50, 99999999999999999999999999999999999999999999999999

_

7:179   08262 99              M9    DC     2, 99

7:180   08263 @                     DC     1, @

7:181

_

7:182   08313 000000..000000        DC    50, 0

7:183   08314 000000..000000        DSC   50, 0

_

7:184   08365 00              MWK1  DC     2, 0

7:185   08366 @                     DC     1, @

7:186

_

7:187   08368 00              MZ2    DC     2, 0

_

7:188   08373 00000           MZ5    DC     5, 0

_

7:189   08423 000000..000000         DC    50, 0,,,         100 ZEROS

7:190   08424 000000..000000  MZ100  DSC   50, 0

7:191

7:192

_

7:193   08478 00099           PAREA DC     5, 00099,,       MACHINE PRODUCT AREA

_

7:194   08483 00100           PAR1  DC     5, 00100,,

_

7:195   08488 00099           PARW  DC     5, 00099,,       MACHINE PRODUCT AREA

7:196

_

7:197   08493 00000           MPMAA DC     5, 0,,,          HIGH ORDER ADDRESS OF MPMA

7:198

_

7:199   08498 00000           M23A1 DC     5, 0,,,          ARG1

_

7:200   08503 00000           M23A2 DC     5, 0,,,          ARG2

7:201

_

7:202   08553 000000..000000        DC    50, 0

7:203   08554 000000..000000        DSC   50, 0

_

7:204   08605 00              M23D1 DC     2, 0 ,,,         FMUL CALLING PROGRAM PRODUCT

7:205   08606 @                     DC     1, @

7:206

_

7:207   08607 0               M23RC DC     1, 0,,,          RETURN CODE

------ 8: fp50-subroutines.sps --------------------------------------------------

8:1                           * ==========================================================

8:2                           *     SET SENSE SWITCHES SUBROUTINE

8:3                           * ==========================================================

8:4

8:5     08612    00005              DS    5,,,             RETURN ADDRESS FROM BTM

8:6     08614 41 00000 00000  SETS1 NOP

8:7                           * CALL SUBROUTINE PRINT SENSE SWITCH STATUS

_

8:8     08626 17 08704 08638        BTM   SWSTA, *+12,,     TRANSMIT RETURN ADDRESS

8:9     08638 34 00000 00102        RCTY

8:10

8:11                          * SET SENSE SWITCHES AND CONTINUE

8:12    08650 39 09341 00100        WATY  SWMS5

8:13    08662 34 00000 00102        RCTY

8:14    08674 48 00000 00000        H

8:15

_

8:16    08686 49 08613 00000        B     SETS1-1,,6,       RETURN

8:17

8:18

8:19                          * ---------------------------------------------

8:20                          * SUBROUTINE PRINT SENSE SWITCH STATUS

8:21                          *

8:22                          *SWSTA

8:23                          * ---------------------------------------------

8:24

8:25    08702    00005              DS    5,,         SWATA RETURN ADDRESS

8:26    08704 41 00000 00000  SWSTA NOP

8:27    08716 34 00000 00102        RCTY

8:28    08728 39 09185 00100        WATY  SWMS2

8:29    08740 34 00000 00102        RCTY

8:30

8:31    08752 39 09245 00100        WATY  SWMS3

8:32    08764 34 00000 00102        RCTY

8:33

8:34    08776 39 09291 00100        WATY  SWMS4

8:35    08788 34 00000 00102        RCTY

8:36

8:37    08800 39 09065 00100        WATY  SWST,,,     SENSE SWITCH STATUS

8:38    08812 34 00000 00102        RCTY

8:39    08824 39 09105 00100        WATY  SWMS1

8:40    08836 34 00000 00102        RCTY

8:41

8:42    08848 46 08884 00100        BC1   *+36,,,      SW1

8:43    08860 39 09169 00100        WATY  OFF

8:44    08872 49 08896 00000        B     *+24

8:45    08884 39 09153 00100        WATY  ON

8:46

8:47    08896 46 08932 00200        BC2   *+36,,,      SW2

8:48    08908 39 09169 00100        WATY  OFF

8:49    08920 49 08944 00000        B     *+24

8:50    08932 39 09153 00100        WATY  ON

8:51

8:52    08944 46 08980 00300        BC3   *+36,,,      SW3

8:53    08956 39 09169 00100        WATY  OFF

8:54    08968 49 08992 00000        B     *+24

8:55    08980 39 09153 00100        WATY  ON

8:56

8:57    08992 46 09028 00400        BC4   *+36,,,      SW4

8:58    09004 39 09169 00100        WATY  OFF

8:59    09016 49 09040 00000        B     *+24

8:60    09028 39 09153 00100        WATY  ON

8:61

8:62    09040 34 00000 00102        RCTY

8:63

_

8:64    09052 49 08703 00000        B     SWSTA-1,,6,         RETURN TO CALLING PROGRAM

8:65

8:66

_

8:67    09065 624555..64620@  SWST  DAC   20,SENSE SWITCH STATUS@

8:68

_

8:69    09105 007100..00740@  SWMS1 DAC   24, 1      2      3      4@

_

8:70    09153 565500..00000@  ON    DAC    8,ON     @

_

8:71    09169 564646..00000@  OFF   DAC    8,OFF    @

8:72

_

8:73    09185 006266..56590@  SWMS2 DAC   30, SW1 ON  = DO NOT PRINT ERROR@

_

8:74    09245 006266..62630@  SWMS3 DAC   23, SW2 ON  = REPEAT TEST@

_

8:75    09291 006266..56590@  SWMS4 DAC   25, SW3 ON  = HALT ON ERROR@

_

8:76    09341 624563..00000@  SWMS5 DAC   43,SET SENSE SWITCHES THEN CONTINUE@

_

8:77    09427 594559..00000@  SWMS6 DAC   43,RERUN MODE. SW2 OFF = EXIT RERUN@

8:78

8:79

8:80

8:81

8:82                          * ---------------------------------------------

8:83                          * SUBROUTINE FL COMMAND TESTS ERROR ROUTINE

8:84                          * ERRC = ERTN(FL-CMD-TEST-STAT)

8:85                          *

8:86                          * CALLING

8:87                          *      BTM   ERTN, *+23,, TRANSMIT ADDRESS OF NEXT INSTRUCTION

8:88                          *      DSA   FL-CMD-TEST-STAT, ESTAT

8:89                          *      NEXT INSTRUCTION

8:90                          *

8:91                          * ARGS

8:92                          *   FL-CMD-TEST-STAT IS EA1

8:93                          *

8:94                          * RETURN CODE

8:95                          *   ERRC    0 = RERUN TEST   1 = GOTO NEXT TEST

8:96                          *

8:97                          * ERROR ROUTINE

8:98                          * SW1 ON  = DO NOT PRINT ERROR

8:99                          * SW2 ON  = REPEAT TEST

8:100                         * SW3 ON  = HALT ON ERROR

8:101                         *

8:102                         *ERTN

8:103                         * ---------------------------------------------

8:104

8:105   09516    00005              DS    5,,         ERTN RETURN ADDRESS

8:106   09518 41 00000 00000  ERTN  NOP

8:107

8:108

8:109                         * GET FIRST ARGUMENT

8:110   09530 26 09565 09517        TF    *+35, ERTN-1,,

_

8:111   09542 12 09565 00006        SM    *+23, 6,,

8:112   09554 26 09577 00000        TF    *+23, 0,,

8:113   09566 26 09914 00000        TF    EA1, 0,,          FL-CMD-TEST-STAT 1=PASS 0=FAIL

8:114

8:115   09578 25 09915 14953        TD    ERRC, FALSE,,     RET CD DEFAULT IS RERUN TEST

8:116

8:117   09590 43 09662 09914        BD    ERPAS, EA1,,      CHECK PASS OR FAIL

8:118

8:119   09602 46 09626 00100        BC1   ERSW3

8:120   09614 39 14957 00100        WATY  FAIL

8:121

8:122   09626 47 09686 00300  ERSW3 BNC3  ERSW2

8:123   09638 48 00000 00000        H     00000, 00000

8:124   09650 49 09686 00000        B     ERSW2

8:125

8:126   09662 41 00000 00000  ERPAS NOP 00000, 00000

8:127   09674 39 14991 00100        WATY  PASS

8:128

8:129   09686 41 00000 00000  ERSW2 NOP

8:130   09698 46 09734 00200        BC2   ERPSW,,,          IF SW2 ON REPEAT TEST

8:131   09710 25 09915 14952        TD    ERRC, TRUE,,      GOTO NEXT TEST

8:132   09722 49 09842 00000        B     ERRT,,,           RETURN

8:133

8:134                         * CALL SUBROUTINE PRINT SENSE SWITCH STATUS

_

8:135   09734 17 08704 09746  ERPSW BTM   SWSTA, *+12,,     TRANSMIT RETURN ADDRESS

8:136   09746 34 00000 00102        RCTY

8:137

8:138                         * SET SENSE SWITCHES AND CONTINUE

8:139   09758 34 00000 00102        RCTY

8:140   09770 34 00000 00102        RCTY

8:141   09782 39 09427 00100        WATY  SWMS6

8:142   09794 34 00000 00102        RCTY

8:143   09806 39 09341 00100        WATY  SWMS5

8:144   09818 34 00000 00102        RCTY

8:145   09830 48 00000 00000        H

8:146

8:147

8:148   09842 41 00000 00000  ERRT  NOP

8:149                         * MOVE RETURN CODE  TO THIRD ARGUMENT

8:150   09854 26 09889 09517        TF    *+35, ERTN-1,,

_

8:151   09866 12 09889 00001        SM    *+23, 1,,

8:152   09878 26 09896 00000        TF    *+18, 0,,

8:153   09890 25 00000 09915        TD    0, ERRC,,

8:154

8:155                         * RETURN

_

8:156   09902 49 09517 00000        B     ERTN-1,,6,                RETURN TO CALLING PROGRAM

8:157

8:158                         * =============

_

8:159   09914 0               EA1   DC    1, 0,,,     FL-CMD-TEST-STAT

_

8:160   09915 0               ERRC  DC    1, 0,,,     ERROR RTN RETURN CODE 0 = RERUN TEST   1 = GOTO NEXT TEST

8:161

8:162

8:163                         * ---------------------------------------------

8:164                         * SUBROUTINE  STATUS = CF(FP1, FP2)

8:165                         *                      COMPARE FLOATING

8:166                         *

8:167                         * COMPARES TWO FLOATING POINT NUMBERS BY

8:168                         *    COMPARING THE MANTISSA FIELDS AND EXPONENT FIELDS

8:169                         *

8:170                         * CALLING

8:171                         *      BTM   CF, *+28,, TRANSMIT ADDRESS OF NEXT INSTRUCTION

8:172                         *      DSA   FP1, FP2, STATUS

8:173                         *      NEXT INSTRUCTION

8:174                         *

8:175                         * CFA1   ADDRESS OF FLOATING POINT NUMBER 1

8:176                         *

8:177                         * CFA2   ADDRESS OF FLOATING POINT NUMBER 2

8:178                         *

8:179                         * CFRC  ADDRESS OF RETURN CODE (STATUS)

8:180                         *            TRUE = EQ, FALSE = NE

8:181                         *

8:182                         * RETURN ADDRESS (BTM)  CF-1

8:183                         *

8:184                         *CF

8:185                         * ---------------------------------------------

8:186

8:187   09920    00005              DS    5,,                     CF RETURN ADDRESS

8:188   09922 41 00000 00000  CF    NOP   00000, 00000

8:189

8:190                         * GET FIRST ARGUMENT

8:191   09934 26 09969 09921        TF    *+35, CF-1,,

_

8:192   09946 12 09969 00012        SM    *+23, 12,,

8:193   09958 26 09981 00000        TF    *+23, 0,,

_

8:194   09970 16 10310 00000        TFM   CFA1,  0,,

8:195   09982 44 10030 09981        BNF   CFSEC, *-1,,      IF NOT INDIRECT GET 2ND ARG

8:196   09994 26 10029 09981        TF    *+35, *-13

8:197   10006 33 10029 00000        CF    *+23

8:198   10018 26 10310 00000        TF    CFA1, 0

8:199

8:200                         * GET SECOND ARGUMENT

8:201   10030 26 10065 09921  CFSEC TF    *+35, CF-1,,

_

8:202   10042 12 10065 00007        SM    *+23, 7,,

8:203   10054 26 10077 00000        TF    *+23, 0,,

_

8:204   10066 16 10315 00000        TFM   CFA2,  0,,

8:205   10078 44 10126 10077        BNF   CFINI, *-1,,      IF NOT INDIRECT GOTO INIT RC

8:206   10090 26 10125 10077        TF    *+35, *-13

8:207   10102 33 10125 00000        CF    *+23

8:208   10114 26 10315 00000        TF    CFA2, 0

8:209

8:210

8:211                         * INITIALIZATION

8:212   10126 25 10316 14952  CFINI TD    CFRC, TRUE,,            SET RETURN CODE

8:213

8:214                         * COMPARE EXPONENTS

_     _

8:215   10138 24 10310 10315        C     CFA1, CFA2, 611,          COMPARE EXPS

8:216   10150 46 10174 01200        BE    MANT,,,                 EXPS ARE EQUAL GOTO MANTISSA

8:217   10162 25 10316 14953        TD    CFRC, FALSE,,           EXPS ARE NE SET RETURN CODE

8:218

8:219                         * GET MANTISSA ADDRESSES

_

8:220   10174 12 10310 00002  MANT  SM    CFA1, 00002,,           BACKUP 2 TO GET MANTISSA OF A

_

8:221   10186 12 10315 00002        SM    CFA2, 00002,,           BACKUP 2 TO GET MANTISSA OF B

8:222

8:223                         * COMPARE MANTASSAS

_     _

8:224   10198 24 10310 10315        C     CFA1, CFA2,611

8:225   10210 46 10234 01200        BE    CFRT,,,                 RETURN IF EQ

8:226   10222 25 10316 14953        TD    CFRC,  FALSE,,          SET RETURN CODE

8:227

8:228   10234 41 00000 00000  CFRT  NOP

8:229                         * MOVE RETURN CODE (CFRC) TO THIRD ARGUMENT

8:230   10246 26 10281 09921        TF    *+35, CF-1,,

_

8:231   10258 12 10281 00002        SM    *+23, 2,,

8:232   10270 26 10288 00000        TF    *+18, 0,,

8:233   10282 25 00000 10316        TD    0, CFRC,,

8:234

8:235                         * RETURN

_

8:236   10294 49 09921 00000        B     CF-1,,6,                RETURN TO CALLING PROGRAM

8:237

8:238                         * =============

_

8:239   10310 00000           CFA1  DC    5, 0

_

8:240   10315 00000           CFA2  DC    5, 0

_

8:241   10316 0               CFRC  DC    1, 0,,,                 RETURN CODE 1=TRUE 0=FALSE

8:242

8:243

------ 9: fp60-symbols-dend.sps --------------------------------------------------

9:1                           * ======================================================

9:2                           * ======================================================

9:3                           * ------------------------------------------------------

9:4                           *SYMBOLS

9:5                           * ------------------------------------------------------

9:6                           * ======================================================

9:7

_

9:8     10319 465356..59630@  TITLE DAC   35,FLOATING POINT DIAGNOSTICS - START@

_

9:9     10389 465356..55440@  DONE  DAC   33,FLOATING POINT DIAGNOSTICS - END@

9:10

9:11

9:12                          * WORK AREAS

9:13

_

9:14    10503 000000..000000        DC    50, 0

9:15    10504 000000..000000        DSC   50, 0

9:16    10554 00              WK1Z  DSC   2,  0,,,    TO ZERO WK1

9:17    10556 @                     DC    1,@

9:18

9:19    10557 000000..000000        DSC   50, 0

9:20    10607 000000..000000        DSC   50, 0

_

9:21    10658 00              WK1   DC    2,  0

9:22    10659 @                     DC    1,@

9:23

9:24    10660 000000..000000        DSC   50, 0

9:25    10710 000000..000000        DSC   50, 0

_

9:26    10761 00              WK2   DC    2,  0

9:27    10762 @                     DC    1,@

9:28

9:29                          *TFL, BTFL, FSR, FSL  DATA START

9:30

9:31

_

9:32    10767 00000           TFINC DC    5, 0,,      TABLE POINTER

9:33

9:34                          * TABLE POINTER

9:35    10777    00010        TFTBP DS    10

9:36

9:37                          * LIST OF  DATA TABLES

_    _

9:38    10782 1080210837      TFTBL DSA   TFTBI, TFTBD

9:39

9:40

9:41    10792    00005        TFI   DS    5

9:42    10797    00005        TFD   DS    5

9:43

9:44

9:45                          * TF TEST IDENTIFICATION MESSAGES

_    _   _

9:46    10802 108691..100000  TFTBI DSA   TFI1, TFI2, TFI3, TFI4, TFI5,

_

9:47    10832 00000                 DSA   00000

9:48

9:49                          * TF TEST DATA TABLE

_    _   _

9:50    10837 110351..700000  TFTBD DSA   TFD1, TFD2, TFD3, TFD4, TFD5,

_

9:51    10867 00000                 DSA   00000

9:52

9:53

9:54                          * TF TEST DATA

9:55

_

9:56    10869 757000..79000@  TFI1  DAC   33,50 DIGIT NUMBER -9999...99  -99 @

9:57

9:58    10934 000000..000000        DSC   50, 0

_            _

9:59    11033 999999..999999        DC    50, -99999999999999999999999999999999999999999999999999

__

9:60    11035 99              TFD1  DC     2, -99

9:61    11036 @                     DC     1, @

9:62

_

9:63    11039 757000..63000@  TFI2  DAC   49,50 DIGIT NEGATIVE NUMBER WITH NEGATIVE EXPONENT @

9:64

9:65    11136 000000..000000        DSC   50, 0

_            _

9:66    11235 123456..999999        DC    50, -12345678901234567890123459999999999999999999999999

__

9:67    11237 04              TFD2  DC     2, -04

9:68    11238 @                     DC     1, @

9:69

_

9:70    11241 757000..59000@  TFI3  DAC   17,50 DIGIT NUMBER @

9:71

9:72    11274 000000..000000        DSC   50, 0

_

9:73    11373 222222..567898        DC    50, 22222222222222222222222222222222222222221234567898

_

9:74    11375 02              TFD3  DC     2, 02

9:75    11376 @                     DC     1, @

9:76

9:77

_

9:78    11379 757000..79000@  TFI4  DAC   29,50 DIGIT NUMBER - EXP = -99 @

9:79

9:80    11436 000000..000000        DSC   50, 0

_

9:81    11535 000000..000000        DC    50, 00000000000000000000000000000000000000000000000000

__

9:82    11537 99              TFD4  DC     2, -99

9:83    11538 @                     DC     1, @

9:84

9:85

9:86

9:87                          *              123456789 123456789 123456789 123456789 1234567890

_

9:88    11541 757000..00000@  TFI5  DAC   33,50 DIGIT NUMBER 9999...99  99 @

9:89

9:90    11606 000000..000000        DSC   50, 0

_

9:91    11705 999999..999999        DC    50, 99999999999999999999999999999999999999999999999999

_

9:92    11707 99              TFD5  DC     2, 99

9:93    11708 @                     DC     1, @

9:94

9:95

9:96

9:97                          *TFL, BTFL, FSR, FSL  DATA END

9:98

9:99

9:100                         *FADD TEST DATA START

9:101

9:102

9:103                         * FADD TESTS TABLE

9:104

9:105

9:106                         * NUMBER OF ELEMENTS IN TABLES

9:107                         * TABLE POINTER

_

9:108   11713 00000           FAINC DC    5, 0,,      TABLE POINTER

9:109

9:110                         * TABLE POINTER

9:111   11728    00015        ASTBL DS    15

9:112

9:113                         * LIST OF FADD DATA TABLES

_    _   _

9:114   11733 117781..311888  FATBL DSA   FATBI, FATBP, FATBQ

9:115

9:116                         * LIST OF FSUB DATA TABLES

_    _   _

9:117   11748 119431..812053  FSTBL DSA   FSTBI, FSTBP, FSTBQ

9:118

9:119

9:120   11763    00005        FAI   DS    5

9:121   11768    00005        FAP   DS    5

9:122   11773    00005        FAQ   DS    5

9:123

9:124                         * FADD TEST IDENTIFICATION MESSAGES

_    _   _

9:125   11778 122091..713439  FATBI DSA   FAI1, FAI2, FAI3, FAI4, FAI5, FAI7, FAI8, FAI9, FAI10, FAI11

_

9:126   11828 00000                 DSA   00000

9:127

9:128                         * P DATA

_    _   _

9:129   11833 123121..013542  FATBP DSA   FAP1, FAP2, FAP3, FAP4, FAP5, FAP7, FAP8, FAP9, FAP10, FAP11

_

9:130   11883 00000                 DSA   00000

9:131

9:132                         * Q DATA

_    _   _

9:133   11888 123181..613548  FATBQ DSA   FAQ1, FAQ2, FAQ3, FAQ4, FAQ5, FAQ7, FAQ8, FAQ9, FAQ10, FAQ11

_

9:134   11938 00000                 DSA   00000

9:135

9:136

9:137                         ********** FSUB TEST DATA 7 AND 8 DIFFER -- P NEGATIVE

9:138                         * FSUB TEST IDENTIFICATION MESSAGES

_    _   _

9:139   11943 122091..713439  FSTBI DSA   FAI1, FAI2, FAI3, FAI4, FAI5, FSI7, FSI8, FAI9, FAI10, FAI11

_

9:140   11993 00000                 DSA   00000

9:141

9:142                         * P DATA

_    _   _

9:143   11998 123121..013542  FSTBP DSA   FAP1, FAP2, FAP3, FAP4, FAP5, FSP7, FSP8, FAP9, FAP10, FAP11

_

9:144   12048 00000                 DSA   00000

9:145

9:146                         * Q DATA

_    _   _

9:147   12053 123181..613548  FSTBQ DSA   FAQ1, FAQ2, FAQ3, FAQ4, FAQ5, FSQ7, FSQ8, FAQ9, FAQ10, FAQ11

_

9:148   12103 00000                 DSA   00000

9:149

9:150

9:151

9:152                         * FADD DATA SYMBOLS

9:153

_

9:154   12153 000000..000000        DC    50, 0

9:155   12154 000000..000000        DSC   50, 0

_

9:156   12205 00              FASUM DC     2, 0

9:157   12206 @                     DC     1, @

9:158

9:159                         * REF  1. 227-5630-1 IBM 1620 FLOATING POINT FEATURE CE MANUAL 1962

9:160                         *     EXAMPLES 1-5 EXPONENT CONFIGURATION

9:161                         *     EXAMPLES 6-11 FRACTION (MANTISSA) CONFIGURATION

9:162                         *     EXAMPLES 1 AND 6 SAME

9:163

9:164

_

9:165   12209 456771..64630@  FAI1  DAC   50,EX1 PE = QE / D = 0 NO NORMALIZING - NO CARRY OUT@

9:166

_

9:167   12310 123                   DC     3, 123

_

9:168   12312 04              FAP1  DC     2, 04

9:169   12313 @                     DC     1, @

9:170

_

9:171   12316 246                   DC     3, 246

_

9:172   12318 04              FAQ1  DC     2, 04

9:173   12319 @                     DC     1, @

9:174

9:175

_

9:176   12321 456772..43490@  FAI2  DAC   50,EX2 PE GT QE - D LT L QF SHIFTED RT TO ALIGN DECI@

9:177

_

9:178   12422 123                   DC     3, 123

_

9:179   12424 06              FAP2  DC     2, 06

9:180   12425 @                     DC     1, @

9:181

_

9:182   12428 246                   DC     3, 246

_

9:183   12430 04              FAQ2  DC     2, 04

9:184   12431 @                     DC     1, @

9:185

9:186

_

9:187   12433 456773..00000@  FAI3  DAC   50,EX3 PE GT QE - D GT L - Q DISCARDED-RESULT IS P  @

9:188

_

9:189   12534 123                   DC     3, 123

_

9:190   12536 08              FAP3  DC     2, 08

9:191   12537 @                     DC     1, @

9:192

_

9:193   12540 246                   DC     3, 246

_

9:194   12542 03              FAQ3  DC     2, 03

9:195   12543 @                     DC     1, @

9:196

9:197

_

9:198   12545 456774..49540@  FAI4  DAC   50,EX4 PE LT QE- D LT L PF SHIFTED RT TO ALIGN DECIM@

9:199

_

9:200   12646 123                   DC     3, 123

_

9:201   12648 02              FAP4  DC     2, 02

9:202   12649 @                     DC     1, @

9:203

_

9:204   12652 246                   DC     3, 246

_

9:205   12654 04              FAQ4  DC     2, 04

9:206   12655 @                     DC     1, @

9:207

9:208

_

9:209   12657 456775..00000@  FAI5  DAC   50,EX5 PE LT QE - D GT L P DISCARDED-RESULT IS Q    @

9:210

_

9:211   12758 123                   DC     3, 123

_

9:212   12760 03              FAP5  DC     2, 03

9:213   12761 @                     DC     1, @

9:214

_

9:215   12764 246                   DC     3, 246

_

9:216   12766 08              FAQ5  DC     2, 08

9:217   12767 @                     DC     1, @

9:218

9:219

9:220                         * EX 6 NO NORMALIZING REQUIRED---NO CARRY OUT -- SAME AS EX1

9:221

_

9:222   12769 456777..63000@  FAI7  DAC   50,EX7 NORMALIZING SHIFT RT - CARRY OUT- EXP ADJUST @

9:223

_

9:224   12870 423                   DC     3, 423

_

9:225   12872 04              FAP7  DC     2, 04

9:226   12873 @                     DC     1, @

9:227

_

9:228   12876 745                   DC     3, 745

_

9:229   12878 04              FAQ7  DC     2, 04

9:230   12879 @                     DC     1, @

9:231

9:232

_

9:233   12881 456777..63000@  FSI7  DAC   49,EX7 (SUB) NORM SHIFT RT - CARRY OUT- EXP ADJUST @

9:234

9:235                         * FSUB TEST - P IS NEGATIVE

9:236

_ _

9:237   12980 423                   DC     3, -423

_

9:238   12982 04              FSP7  DC     2, 04

9:239   12983 @                     DC     1, @

9:240

_

9:241   12986 745                   DC     3, 745

_

9:242   12988 04              FSQ7  DC     2, 04

9:243   12989 @                     DC     1, @

9:244

9:245

_

9:246   12991 456778..55460@  FAI8  DAC   50,EX8 NORM SHIFT RT-CARRY OT CAUSES EXP OV MACH INF@

9:247

_

9:248   13092 423                   DC     3, 423

_

9:249   13094 99              FAP8  DC     2, 99

9:250   13095 @                     DC     1, @

9:251

_

9:252   13098 745                   DC     3, 745

_

9:253   13100 99              FAQ8  DC     2, 99

9:254   13101 @                     DC     1, @

9:255

_

9:256   13103 456778..55460@  FSI8  DAC   50,EX8 (SUB) NORM SHIFT RT-CARRY OT- EXP OV MACH INF@

9:257

9:258                         * FSUB TEST - P IS NEGATIVE

9:259

_ _

9:260   13204 423                   DC     3, -423

_

9:261   13206 99              FSP8  DC     2, 99

9:262   13207 @                     DC     1, @

9:263

_

9:264   13210 745                   DC     3, 745

_

9:265   13212 99              FSQ8  DC     2, 99

9:266   13213 @                     DC     1, @

9:267

_

9:268   13215 456779..79790@  FAI9  DAC   50,EX9 ZERO FRACTION RESULT - MACHINE ZERO 0...0 -99@

9:269

_

9:270   13316 345                   DC     3, 345

_

9:271   13318 04              FAP9  DC     2, 04

9:272   13319 @                     DC     1, @

9:273

_ _

9:274   13322 345                   DC     3, -345

_

9:275   13324 04              FAQ9  DC     2, 04

9:276   13325 @                     DC     1, @

9:277

9:278

_

9:279   13327 456771..44510@  FAI10 DAC   50,EX10 NORMALIZING SHIFT LEFT - SHIFT LEFT- EXP ADJ@

9:280

_

9:281   13428 345                   DC     3, 345

_

9:282   13430 04              FAP10 DC     2, 04

9:283   13431 @                     DC     1, @

9:284

_ _

9:285   13434 321                   DC     3, -321

_

9:286   13436 04              FAQ10 DC     2, 04

9:287   13437 @                     DC     1, @

9:288

9:289

_

9:290   13439 456771..56000@  FAI11 DAC   50,EX11 NORMALIZING SHIFT LEFT- EXP OV MACHINE ZERO @

9:291

_

9:292   13540 345                   DC     3, 345

__

9:293   13542 99              FAP11 DC     2, -99

9:294   13543 @                     DC     1, @

9:295

_ _

9:296   13546 321                   DC     3, -321

__

9:297   13548 99              FAQ11 DC     2, -99

9:298   13549 @                     DC     1, @

9:299

9:300                         *

9:301

9:302                         *FADD TEST DATA END --------------------

9:303

9:304

9:305                         *FMUL TEST DATA START

9:306

_

9:307   13551 333333..33330@  FMULT DAC   45,====== FLOATING MULTIPLY (FMUL) TESTS ======@

9:308

9:309                         * FMUL TESTS TABLE

9:310

_

9:311   13644 00000           FMINC DC    5, 0,,      TABLE POINTER

9:312

9:313                         * TABLE POINTER

9:314   13659    00015        FMTP  DS    15

9:315

9:316                         * LIST OF TABLES

_    _   _

9:317   13664 136941..413774  FMTBL DSA   FMTBI, FMTBP, FMTBQ

9:318

9:319   13679    00005        FMI   DS    5

9:320   13684    00005        FMP   DS    5

9:321   13689    00005        FMQ   DS    5

9:322

9:323

9:324                         * FMUL TEST IDENTIFICATION MESSAGES

_    _   _

9:325   13694 142151..114871  FMTBI DSA   FMI1, FMI10, FMI11, FMI2, FMI3, FMI4, FMI5

_

9:326   13729 00000                 DSA   00000

9:327

9:328                         *MULTIPLICANDS

_    _   _

9:329   13734 143181..214944  FMTBP DSA   FMP1, FMP10, FMP11, FMP2, FMP3, FMP4, FMP5

_

9:330   13769 00000                 DSA   00000

9:331

9:332                         *MULTIPLIERS

_    _   _

9:333   13774 143241..814950  FMTBQ DSA   FMQ1, FMQ10, FMQ11, FMQ2, FMQ3, FMQ4, FMQ5

_

9:334   13809 00000                 DSA   00000

9:335

9:336

9:337                         * FMUL DATA SYMBOLS

9:338

_

9:339   13859 000000..000000        DC    50, 0

9:340   13860 000000..000000        DSC   50, 0

_

9:341   13911 00              FMPRD DC     2, 0

9:342   13912 @                     DC     1, @

9:343

9:344

9:345                         * REF  227-5630-1 IBM 1620 FLOATING POINT FEATURE CE MANUAL 1962

9:346                         *      --------------------------------------------------------

9:347

9:348                         *              123456789 123456789 123456789 123456789 1234567890

_

9:349   13915 717070..04000@  FMI0  DAC   47,100 DIGIT NUMBER TEST +MP,-MQ, +PE,-MQ (FMUL) @

9:350                         *               123456789 123456789 123456789 123456789 123456789

_

9:351   14057 123456..222222        DC    50, 12345678222222222222222222222222222222222222222222

9:352   14058 222222..222222        DSC   50, 22222222222222222222222222222222222222222222222222

_

9:353   14109 99              FMP0  DC     2, 99

9:354   14110 @                     DC     1, @

9:355

9:356                         *               123456789 123456789 123456789 123456789 123456789

_

9:357   14160 123456..333333        DC    50, 12345669333333333333333333333333333333333333333333

9:358   14161 333333..333333        DSC   49, 3333333333333333333333333333333333333333333333333

_

9:359   14210 3                     DSC    1, -3

__

9:360   14212 50              FMQ0  DC     2, -50

9:361   14213 @                     DC     1, @

9:362

9:363

9:364                         * FMUL EXAMPLE 1. NO EXPONENT MODIFY REQUIRED

9:365

9:366                         *              123456789 123456789 123456789 123456789 1234567890

_

9:367   14215 456771..04000@  FMI1  DAC   50,EX1A NO EXP MOD REQUIRED +MP,+MQ, +EP,+EQ (FMUL) @

9:368

_

9:369   14316 345                   DC     3, 345

_

9:370   14318 07              FMP1  DC     2, 07

9:371   14319 @                     DC     1, @

9:372

_

9:373   14322 432                   DC     3, 432

_

9:374   14324 03              FMQ1  DC     2, 03

9:375   14325 @                     DC     1, @

9:376

9:377

_

9:378   14327 456771..04000@  FMI10 DAC   50,EX1B NO EXP MOD REQUIRED -MP,+MQ, -EP,+EQ (FMUL) @

9:379

_ _

9:380   14428 345                   DC     3, -345

__

9:381   14430 07              FMP10 DC     2, -07

9:382   14431 @                     DC     1, @

9:383

_

9:384   14434 432                   DC     3, 432

_

9:385   14436 03              FMQ10 DC     2, 03

9:386   14437 @                     DC     1, @

9:387

9:388

_

9:389   14439 456771..04000@  FMI11 DAC   50,EX1C NO EXP MOD REQUIRED -MP,-MQ, -EP,-EQ (FMUL) @

9:390

_ _

9:391   14540 345                   DC     3, -345

__

9:392   14542 07              FMP11 DC     2, -07

9:393   14543 @                     DC     1, @

9:394

_ _

9:395   14546 432                   DC     3, -432

__

9:396   14548 03              FMQ11 DC     2, -03

9:397   14549 @                     DC     1, @

9:398

9:399

9:400                         * FMUL EXAMPLE 2. EXPONENT MODIFY REQUIRED

9:401

9:402                         *             123456789 123456789 123456789 123456789 1234567890

_

9:403   14551 456772..04000@  FMI2 DAC   49,EX2 EXPONENT MOD REQ-ED +MP,+MQ, +EP,+EQ (FMUL) @

9:404

_

9:405   14650 123                   DC     3, 123

_

9:406   14652 02              FMP2  DC     2, 02

9:407   14653 @                     DC     1, @

9:408

_

9:409   14656 246                   DC     3, 246

_

9:410   14658 04              FMQ2  DC     2, 04

9:411   14659 @                     DC     1, @

9:412

9:413

9:414                         * FMUL EXAMPLE 3. DIGIT FORCE -- MACHINE INFINITY

9:415

9:416                         *             123456789 123456789 123456789 123456789 1234

_

9:417   14661 456773..04000@  FMI3 DAC   44,EX3 DIGIT FORCE -- MACHINE INFINITY (FMUL) @

9:418

_

9:419   14750 345                   DC     3, 345

_

9:420   14752 43              FMP3  DC     2, 43

9:421   14753 @                     DC     1, @

9:422

_

9:423   14756 432                   DC     3, 432

_

9:424   14758 64              FMQ3  DC     2, 64

9:425   14759 @                     DC     1, @

9:426

9:427

9:428                         * FMUL EXAMPLE 4. EXPONENT OVERFLOW -- MODIFY PE TO 99

9:429

9:430                         *             123456789 123456789 123456789 123456789 123456789

_

9:431   14761 456774..04000@  FMI4 DAC   49,EX4 EXPONENT OVERFLOW -- MODIFY PE TO 99 (FMUL) @

9:432

_

9:433   14860 123                   DC     3, 123

_

9:434   14862 38              FMP4  DC     2, 38

9:435   14863 @                     DC     1, @

9:436

_

9:437   14866 246                   DC     3, 246

_

9:438   14868 62              FMQ4  DC     2, 62

9:439   14869 @                     DC     1, @

9:440

9:441

9:442                         * FMUL EXAMPLE 5. SPECIAL CASE 999 X 199

9:443

9:444                         *             123456789 123456789 123456789 123456789 12345

_

9:445   14871 456775..04000@  FMI5 DAC   35,EX5 SPECIAL CASE 999 X 199 (FMUL) @

9:446

_

9:447   14942 999                   DC     3, 999

_

9:448   14944 01              FMP5  DC     2, 01

9:449   14945 @                     DC     1, @

9:450

_

9:451   14948 199                   DC     3, 199

_

9:452   14950 01              FMQ5  DC     2, 01

9:453   14951 @                     DC     1, @

9:454

9:455

9:456                         *FMUL TEST DATA  END --------------------

9:457

9:458                         * CONSTANTS AND STATUS CODES

_

9:459   14952 1               TRUE  DC    1,1

_

9:460   14953 0               FALSE DC    1,0

9:461

_

9:462   14954 0               ESTAT DC    1,0,,, FL COMMAND TEST ERR RTN STATUS

_

9:463   14955 0               STAT  DC    1,0,,, FL COMMAND TEST STATUS

9:464

_

9:465   14957 333333..33330@  FAIL  DAC   17,==== FAILED ====@

_

9:466   14991 202057..20200@  PASS  DAC   11,--PASSED--@

9:467

9:468

_

9:469   15012 0               OV    DC    1,0,,,      OVERFLOW FLAG

_

9:470   15013 0               XCK   DC    1,0,,,      EXPONENT OVERFLOW FLAG

9:471

_

9:472   15015 005665..66000@  OVM1  DAC   11, OVERFLOW @

_

9:473   15037 004567..66000@  XCKM1 DAC   20, EXPONENT OVERFLOW @

9:474

9:475

9:476                         * -------------------------------------------------

9:477                         * -------------------------------------------------

9:478                         * -------------------------------------------------

9:479   00402                       DEND  FPMAIN

Symbol Cross-Reference Table

============================

Id  Source File

--  --------------------------------------------------

1  fp10-main.sps

2  fp20-tftest.sps

3  fp21-tftest.sps

4  fp30-fadd.sps

5  fp31-fixadd.sps

6  fp40-fmul.sps

7  fp41-fixmul.sps

8  fp50-subroutines.sps

9  fp60-symbols-dend.sps

Symbol   Addr.  Type    Defined  References

-------  -----  ------  -------  -------------------------------------------------------------

A21      03216  <inst>   5:45     4:101    5:49     5:58     5:66     5:74     5:305    5:312

A21A1    06253  DC       5:388    5:62     5:90     5:102    5:106    5:113

A21A2    06258  DC       5:389    5:70     5:91     5:103    5:107

A21D1    06360  DC       5:393    5:77     5:81     5:301

A21RC    06366  DC       5:396    5:301    5:308

A9       05932  DC       5:366    5:272    5:275

ACMF1    04704  <inst>   5:238    5:229    5:230

ACMF2    04752  <inst>   5:242    5:231    5:232

ACO      04596  <inst>   5:229    5:222

ADDM     04104  <inst>   5:165    5:122    5:142    5:161

ADIGT    04404  <inst>   5:204    5:208

ADJP     03780  <inst>   5:129    5:123

ADJP1    03840  <inst>   5:135    5:130

ADJQ     03936  <inst>   5:147    5:124

ADJQ1    04008  <inst>   5:154    5:149

ADSA     03118  DSA      4:102    4:16

AED      05281  DC       5:320    5:95     5:96     5:97     5:98     5:129    5:136    5:148

5:155

AELS     05088  <inst>   5:284    5:259

AEOVM    05667  DAC      5:356

AEP      05403  DC       5:332    5:88     5:90     5:96     5:121    5:151    5:255    5:256

AEPQ     05619  DC       5:348    5:132    5:151    5:223    5:254    5:255    5:258    5:263

5:265    5:284    5:285    5:287    5:293    5:293    5:294

AEQ      05511  DC       5:340    5:89     5:91     5:97     5:121    5:132    5:256    5:258

AFLAG    04368  <inst>   5:200    5:201

AFLG     05286  DC       5:322    5:197    5:206    5:214

AINF     04968  <inst>   5:270    5:264

AINF1    05028  <inst>   5:275    5:271

AINF2    05064  <inst>   5:278    5:270

AL       03696  <inst>   5:115    5:117

ALDON    01146  <inst>   1:154

AMKFP    05148  <inst>   5:293    5:133    5:152    5:224    5:261    5:266    5:279    5:286

AML      05276  DC       5:319    5:112    5:116    5:129    5:139    5:148    5:158    5:191

5:235    5:237    5:239    5:241    5:276

AMP      05397  DSC      5:329    5:104    5:106    5:135    5:137    5:137    5:138    5:141

5:141    5:150    5:169    5:229    5:230    5:231    5:232

5:238    5:238    5:244

AMPA     05296  DC       5:325    5:113    5:115    5:117    5:198    5:200    5:201    5:202

5:204    5:207    5:211

AMPAA    06248  DC       5:386

AMPQ     05613  DSC      5:345    5:131    5:150    5:168    5:169    5:170    5:184    5:190

5:193    5:198    5:210    5:212    5:213    5:215    5:221

5:243    5:244    5:245    5:246    5:246    5:247    5:247

5:270    5:271    5:272    5:273    5:275    5:278    5:288

5:295

AMQ      05505  DSC      5:337    5:105    5:107    5:131    5:154    5:156    5:156    5:157

5:160    5:160    5:170    5:242    5:242    5:245

ANRM     05622  DC       5:352    5:166    5:187    5:196    5:259

AOV      05621  DC       5:351    5:167    5:175    5:233    5:260

AOVM     05625  DAC      5:355

APQFP    05829  DC       5:359

ASF      04464  <inst>   5:210    5:204

ASF2     04524  <inst>   5:215    5:210

ASHFT    04212  <inst>   5:183    5:174

ASTBL    11728  DS       9:111    4:42     4:43     4:44     4:52     4:53     4:54     4:60

4:61     4:64     4:65     4:69     4:70     4:71

AWK1     06034  DC       5:371    5:195    5:212    5:215

AZ100    06142  DSC      5:379

AZ101    06243  DSC      5:383    5:104    5:105    5:168    5:184    5:195    5:213    5:221

5:243    5:288

AZ2      06037  DC       5:374

AZ5      06042  DC       5:375    5:88     5:89     5:95     5:254

AZCK     04548  <inst>   5:221    5:176    5:193

AZCNT    05291  DC       5:323    5:188    5:205    5:284

AZRO     04572  <inst>   5:223    5:185

BTSUB    02392  <inst>   3:18     2:131    3:24

CF       09922  <inst>   8:188    3:23     5:300    7:136    8:191    8:201    8:230    8:236

CFA1     10310  DC       8:239    8:194    8:198    8:215    8:220    8:224

CFA2     10315  DC       8:240    8:204    8:208    8:215    8:221    8:224

CFINI    10126  <inst>   8:212    8:205

CFRC     10316  DC       8:241    8:212    8:217    8:226    8:233

CFRT     10234  <inst>   8:228    8:225

CFSEC    10030  <inst>   8:201    8:195

DONE     10389  DAC      9:9      1:157

EA1      09914  DC       8:159    8:113    8:117

ERPAS    09662  <inst>   8:126    8:117

ERPSW    09734  <inst>   8:135    8:130

ERRC     09915  DC       8:160    8:115    8:131    8:153

ERRT     09842  <inst>   8:148    8:132

ERSW2    09686  <inst>   8:129    8:122    8:124

ERSW3    09626  <inst>   8:122    8:119

ERTN     09518  <inst>   8:106    2:141    4:107    6:74     8:110    8:150    8:156

ESTAT    14954  DC       9:462    2:142    2:144    4:108    4:110    6:75     6:77

EXADJ    04824  <inst>   5:254    5:216    5:234

FACR     05260  <inst>   5:312

FAEND    03186  <inst>   4:113    4:110

FAI      11763  DS       9:120    4:69     4:76

FAI1     12209  DAC      9:165    9:125    9:139

FAI10    13327  DAC      9:279    9:125    9:139

FAI11    13439  DAC      9:290    9:125    9:139

FAI2     12321  DAC      9:176    9:125    9:139

FAI3     12433  DAC      9:187    9:125    9:139

FAI4     12545  DAC      9:198    9:125    9:139

FAI5     12657  DAC      9:209    9:125    9:139

FAI7     12769  DAC      9:222    9:125

FAI8     12991  DAC      9:246    9:125

FAI9     13215  DAC      9:268    9:125    9:139

FAIL     14957  DAC      9:465    8:120

FAINC    11713  DC       9:108    4:32     4:60     4:64     4:65     4:67

FALSE    14953  DC       9:460    2:65     5:166    5:167    5:187    8:115    8:217    8:226

FANXT    02826  <inst>   4:57     4:45     4:115

FAP      11768  DS       9:121    4:70     4:82     4:83     4:84     4:85     4:102

FAP1     12312  DC       9:168    9:129    9:143

FAP10    13430  DC       9:282    9:129    9:143

FAP11    13542  DC       9:293    9:129    9:143

FAP2     12424  DC       9:179    9:129    9:143

FAP3     12536  DC       9:190    9:129    9:143

FAP4     12648  DC       9:201    9:129    9:143

FAP5     12760  DC       9:212    9:129    9:143

FAP7     12872  DC       9:225    9:129

FAP8     13094  DC       9:249    9:129

FAP9     13318  DC       9:271    9:129    9:143

FAQ      11773  DS       9:122    4:71     4:89     4:102

FAQ1     12318  DC       9:172    9:133    9:147

FAQ10    13436  DC       9:286    9:133    9:147

FAQ11    13548  DC       9:297    9:133    9:147

FAQ2     12430  DC       9:183    9:133    9:147

FAQ3     12542  DC       9:194    9:133    9:147

FAQ4     12654  DC       9:205    9:133    9:147

FAQ5     12766  DC       9:216    9:133    9:147

FAQ7     12878  DC       9:229    9:133

FAQ8     13100  DC       9:253    9:133

FAQ9     13324  DC       9:275    9:133    9:147

FARIT    03042  <inst>   4:89     4:12     4:34

FARPT    02946  <inst>   4:74     4:111

FASUM    12205  DC       9:156    4:82     4:84     4:89     4:102

FATBI    11778  DSA      9:125    4:39     9:114

FATBL    11733  DSA      9:114    4:39     4:40     4:41     4:42     4:43     4:44

FATBP    11833  DSA      9:129    4:40     9:114

FATBQ    11888  DSA      9:133    4:41     9:114

FATST    02466  <inst>   4:7      1:137    1:144    4:10     4:14     4:18     4:62

FIRST    00462  <inst>   1:111

FMCR     07720  <inst>   7:145

FMEND    06870  <inst>   6:80     6:77

FMI      13679  DS       9:319    6:40     6:47

FMI0     13915  DAC      9:349

FMI1     14215  DAC      9:367    9:325

FMI10    14327  DAC      9:378    9:325

FMI11    14439  DAC      9:389    9:325

FMI2     14551  DAC      9:403    9:325

FMI3     14661  DAC      9:417    9:325

FMI4     14761  DAC      9:431    9:325

FMI5     14871  DAC      9:445    9:325

FMINC    13644  DC       9:311    6:18     6:31     6:35     6:36     6:38

FML      07748  DC       7:151    7:77     7:81     7:92     7:108    7:109    7:114    7:122

FMNXT    06516  <inst>   6:29     6:81

FMP      13684  DS       9:320    6:41     6:53     6:54     6:55     6:56     6:69

FMP0     14109  DC       9:353

FMP1     14318  DC       9:370    9:329

FMP10    14430  DC       9:381    9:329

FMP11    14542  DC       9:392    9:329

FMP2     14652  DC       9:406    9:329

FMP3     14752  DC       9:420    9:329

FMP4     14862  DC       9:434    9:329

FMP5     14944  DC       9:448    9:329

FMPRD    13911  DC       9:341    6:53     6:55     6:58     6:69

FMQ      13689  DS       9:321    6:42     6:58     6:69

FMQ0     14212  DC       9:360

FMQ1     14324  DC       9:374    9:333

FMQ10    14436  DC       9:385    9:333

FMQ11    14548  DC       9:396    9:333

FMQ2     14658  DC       9:410    9:333

FMQ3     14758  DC       9:424    9:333

FMQ4     14868  DC       9:438    9:333

FMQ5     14950  DC       9:452    9:333

FMRPT    06636  <inst>   6:45     6:78

FMTBI    13694  DSA      9:325    6:22     9:317

FMTBL    13664  DSA      9:317    6:22     6:23     6:24     6:25     6:26     6:27

FMTBP    13734  DSA      9:329    6:23     9:317

FMTBQ    13774  DSA      9:333    6:24     9:317

FMTP     13659  DS       9:314    6:25     6:26     6:27     6:31     6:32     6:35     6:36

6:40     6:41     6:42

FMTST    06372  <inst>   6:8      1:150    6:33

FMULT    13551  DAC      9:307    6:12

FPMAIN   00402  <inst>   1:98     9:479    9:479

FSI7     12881  DAC      9:233    9:139

FSI8     13103  DAC      9:256    9:139

FSP7     12982  DC       9:238    9:143

FSP8     13206  DC       9:261    9:143

FSQ7     12988  DC       9:242    9:147

FSQ8     13212  DC       9:265    9:147

FSTBI    11943  DSA      9:139    4:49     9:117

FSTBL    11748  DSA      9:117    4:49     4:50     4:51     4:52     4:53     4:54

FSTBP    11998  DSA      9:143    4:50     9:117

FSTBQ    12053  DSA      9:147    4:51     9:117

FXAR1    04164  <inst>   5:170    5:52

FXAR2    04788  <inst>   5:245    5:54

M23      06900  <inst>   7:35     6:68     7:38     7:46     7:54     7:140    7:148

M23A1    08498  DC       7:199    7:42     7:66     7:70     7:72     7:78

M23A2    08503  DC       7:200    7:50     7:67     7:71     7:73

M23D1    08605  DC       7:204    7:57     7:61     7:137

M23RC    08607  DC       7:207    7:137    7:143

M9       08262  DC       7:179    7:90     7:91     7:98

MANT     10174  <inst>   8:220    8:216

ME100    07380  <inst>   7:98     7:88

MKFP     07596  <inst>   7:129    7:96

ML       07224  <inst>   7:80     7:82

MPMA     07848  DSC      7:155    7:72     7:102

MPMAA    08493  DC       7:197    7:78     7:80     7:82

MQMA     07949  DSC      7:160    7:73     7:102

MUL      07392  <inst>   7:101    7:87

MWK1     08365  DC       7:184    7:91     7:94     7:95

MZ100    08424  DSC      7:190    7:101

MZ2      08368  DC       7:187

MZ5      08373  DC       7:188    7:65

OFF      09169  DAC      8:71     8:43     8:48     8:53     8:58

ON       09153  DAC      8:70     8:45     8:50     8:55     8:60

OV       15012  DC       9:469

OVM1     15015  DAC      9:472    4:94     6:62

PAR1     08483  DC       7:194    7:107    7:108    7:109    7:110    7:111    7:113

PAREA    08478  DC       7:193    7:101    7:121    7:123

PARW     08488  DC       7:195    7:111    7:112    7:113    7:121    7:122    7:123    7:126

PASS     14991  DAC      9:466    8:127

PQFP     08153  DC       7:170    5:294    5:295    5:301    7:131    7:132    7:137

PQMA     08050  DSC      7:165    7:95     7:126    7:132

PRSH     07548  <inst>   7:121    7:110    7:117

RETAD    02460  DS       3:35     2:130    3:31

SETS1    08614  <inst>   8:6      1:113    8:16

STAT     14955  DC       9:463    2:65     2:135    2:142    3:24     4:102    4:108    6:69

6:75

SUBTB    02754  <inst>   4:49     4:35

SUMEX    08159  DC       7:173    7:65     7:66     7:67     7:86     7:90     7:98     7:116

7:118    7:130    7:130    7:131

SWMS1    09105  DAC      8:69     8:39

SWMS2    09185  DAC      8:73     8:28

SWMS3    09245  DAC      8:74     8:31

SWMS4    09291  DAC      8:75     8:34

SWMS5    09341  DAC      8:76     8:12     8:143

SWMS6    09427  DAC      8:77     8:141

SWST     09065  DAC      8:67     8:37

SWSTA    08704  <inst>   8:26     8:8      8:64     8:135

TBTFL    02112  <inst>   2:126    2:106

TER      02220  <inst>   2:141    2:81     2:101    2:122    2:134

TEXCK    02184  <inst>   2:133    2:71     2:80     2:100    2:121

TFD      10797  DS       9:42     2:53     2:71     2:76     2:77     2:91     2:95     2:107

2:110    2:112    2:118    2:126    2:127

TFD1     11035  DC       9:60     9:50

TFD2     11237  DC       9:67     9:50

TFD3     11375  DC       9:74     9:50

TFD4     11537  DC       9:82     9:50

TFD5     11707  DC       9:92     9:50

TFEND    02266  <inst>   2:147    2:144

TFI      10792  DS       9:41     2:52     2:57

TFI1     10869  DAC      9:56     9:46

TFI2     11039  DAC      9:63     9:46

TFI3     11241  DAC      9:70     9:46

TFI4     11379  DAC      9:78     9:46

TFI5     11541  DAC      9:88     9:46

TFINC    10767  DC       9:32     2:30     2:44     2:48     2:50

TFINS    01284  <inst>   2:13     2:12     2:74     2:89     2:105

TFNXT    01440  <inst>   2:40     2:149

TFRPT    01536  <inst>   2:56     2:145

TFSL     01740  <inst>   2:89     2:75

TFSLC    01860  <inst>   2:99     2:95     2:96     2:97     2:97

TFSR     01896  <inst>   2:105    2:90

TFSRC    02076  <inst>   2:120    2:118    2:119

TFSRF    02040  <inst>   2:117    2:114

TFTBD    10837  DSA      9:50     2:35     9:38

TFTBI    10802  DSA      9:46     2:34     9:38

TFTBL    10782  DSA      9:38     2:34     2:35     2:36     2:37

TFTBP    10777  DS       9:35     2:36     2:37     2:44     2:45     2:48     2:52     2:53

TFTST    01236  <inst>   2:7      1:116    1:121    1:126    1:131    2:10     2:16     2:46

TFWT     01356  <inst>   2:24     2:18

TITLE    10319  DAC      9:8      1:102

TRUE     14952  DC       9:459    2:135    5:175    5:196    8:131    8:212

WK1      10658  DC       9:21     2:66     2:67     2:76     2:79     2:93     2:94     2:94

2:98     2:99     2:109    2:111    2:115    2:116    2:117

2:117    2:120    2:129    2:133    3:24

WK1Z     10554  DSC      9:16     2:66     2:68

WK2      10761  DC       9:26     2:68     2:69

WTITL    02610  <inst>   4:26     4:20

XCK      15013  DC       9:470

XCKM1    15037  DAC      9:473    4:96     6:64

CU03 - Indirect Addressing Diagnostic

//==============================================================================

//

// CU03 - Indirect Addressing Diagnostic

//

// Program Switch settings:

//

//   PS1:   ON - Bypass error type out

//         OFF - Type out routine number on error

//   PS2:   ON - Loop in routine

//         OFF - Continue to next routine

//   PS3:   ON - Stop on error

//         OFF - Do not stop on error, continue

//   PS4:   ON - Repeat test CU03

//         OFF - Run test CU03 once

//

// Check switches settings:

//

//   DISK I/O - STOP

//   PARITY   - STOP

//   I/O      - STOP

//   O'FLOW   - STOP

//

// Start addresses:

//

//   00828 - Full test

//

// Directions:

//

//   1. Load CU03 diagnostic

//   2. Press START

//   3. Press START

//

//==============================================================================

Sample Output – CU03

CS01 - Check Stops Diagnostic

//==============================================================================

//

// CS01 - Check Stops Diagnostic

//

// Program Switch settings:

//

//   PS1:  not used

//   PS2:  not used

//   PS3:  not used

//   PS4:  not used

//

// Check switches settings:

//

//   DISK I/O - STOP

//   PARITY   - STOP

//   I/O      - STOP

//   O'FLOW   - STOP

//

// Start addresses:

//

//   00402 - Full test

//

// Directions:

//

//     1. Load CS01 diagnostic

//     2. Press START

//     3. Verify CHECK STOP & MAR CHK are on and MAR = C/C/C/C/C-8-2

//     4. Press RESET

//     5. Press START

//     6. Verify CHECK STOP & MAR CHK are on and MAR = C/C/C/C-8-2/C

//     7. Press RESET

//     8. Press START

//     9. Verify CHECK STOP & MAR CHK are on and MAR = C/C/C-8-2/C/C

//    10. Press RESET

//    11. Press START

//    12. Verify CHECK STOP & MAR CHK are on and MAR = C/C-8-2/C/C/C

//    13. Press RESET

//    14. Press START

//    15. Verify CHECK STOP & MAR CHK are on and MAR = C-8-2/C/C/C/C

//    16. Press RESET

//    17. Press START

//    18. Verify CHECK STOP & MAR CHK are on and MAR = C/C/C/C/8-2-1

//    19. Press RESET

//    20. Press START

//    21. Verify CHECK STOP & MAR CHK are on and MAR = C/C/C/8-2-1/C

//    22. Press RESET

//    23. Press START

//    24. Verify CHECK STOP & MAR CHK are on and MAR = C/C/8-2-1/C/C

//    25. Press RESET

//    26. Press START

//    27. Verify CHECK STOP & MAR CHK are on and MAR = C/8-2-1/C/C/C

//    28. Press RESET

//    29. Press START

//    30. Verify CHECK STOP & MAR CHK are on and MAR = 8-2-1/C/C/C/C

//    31. Press RESET

//    32. Press START

//    33. Verify CHECK STOP & MAR CHK are on and MAR = C/C/C/C/C-8-4

//    34. Press RESET

//    35. Press START

//    36. Verify CHECK STOP & MAR CHK are on and MAR = C/C/C/C-8-4/C

//    37. Press RESET

//    38. Press START

//    39. Verify CHECK STOP & MAR CHK are on and MAR = C/C/C-8-4/C/C

//    40. Press RESET

//    41. Press START

//    42. Verify CHECK STOP & MAR CHK are on and MAR = C/C-8-4/C/C/C

//    43. Press RESET

//    44. Press START

//    45. Verify CHECK STOP & MAR CHK are on and MAR = C-8-4/C/C/C/C

//    46. Press RESET

//    47. Press START

//    48. Verify CHECK STOP & MAR CHK are on and MAR = C/C/C/C/8-4-1

//    49. Press RESET

//    50. Press START

//    51. Verify CHECK STOP & MAR CHK are on and MAR = C/C/C/8-4-1/C

//    52. Press RESET

//    53. Press START

//    54. Verify CHECK STOP & MAR CHK are on and MAR = C/C/8-4-1/C/C

//    55. Press RESET

//    56. Press START

//    57. Verify CHECK STOP & MAR CHK are on and MAR = C/8-4-1/C/C/C

//    58. Press RESET

//    59. Press START

//    60. Verify CHECK STOP & MAR CHK are on and MAR = 8-4-1/C/C/C/C

//    61. Press RESET

//    62. Press START

//    63. Verify CHECK STOP & MAR CHK are on and MAR = C/C/C/C/8-4-2

//    64. Press RESET

//    65. Press START

//    66. Verify CHECK STOP & MAR CHK are on and MAR = C/C/C/8-4-2/C

//    67. Press RESET

//    68. Press START

//    69. Verify CHECK STOP & MAR CHK are on and MAR = C/C/8-4-2/C/C

//    70. Press RESET

//    71. Press START

//    72. Verify CHECK STOP & MAR CHK are on and MAR = C/8-4-2/C/C/C

//    73. Press RESET

//    74. Press START

//    75. Verify CHECK STOP & MAR CHK are on and MAR = 8-4-2/C/C/C/C

//    76. Press RESET

//    77. Press START

//    78. Verify CHECK STOP & MAR CHK are on and MAR = C/C/C/C/C-8-4-2-1

//    79. Press RESET

//    80. Press START

//    81. Verify CHECK STOP & MAR CHK are on and MAR = C/C/C/C-8-4-2-1/C

//    82. Press RESET

//    83. Press START

//    84. Verify CHECK STOP & MAR CHK are on and MAR = C/C/C-8-4-2-1/C/C

//    85. Press RESET

//    86. Press START

//    87. Verify CHECK STOP & MAR CHK are on and MAR = C/C-8-4-2-1/C/C/C

//    88. Press RESET

//    89. Press START

//    90. Verify CHECK STOP & MAR CHK are on and MAR = C-8-4-2-1/C/C/C/C

//    91. Press RESET

//    92. Press START

//    93. Verify CHECK STOP is on and OPERATION REGISTER = 7/7

//    94. Press RESET

//    95. Press START

//    96. Verify CHECK STOP is on and OPERATION REGISTER = C/C-8-4-2-1

//    97. Press RESET

//    98. Press START

//    99. Verify CHECK STOP is on and OPERATION REGISTER = C-8-4-2-1/C

//   100. Press RESET

//   101. Press START

//   102. Verify CHECK STOP & MAR CHK are on and MAR = C/8/4/2/1

//   103. Press RESET

//   104. Press INSERT

//   105. Press RELEASE

//   106. Press START

//   107. Verify CHECK STOP & MAR CHK are on and MAR = C-4-2/C/C/C/C

//   108. Press RESET

//   109. Press START

//   110. Verify CHECK STOP & MAR CHK are on and MAR = C-8-1/C-8-1/C-8-1/C-8-1/C-8-1

//   111. Press RESET

//   112. Press START

//   113. Type A

//   114. Verify CHECK STOP & MAR CHK are on and MAR = 1/C/C/C/C

//   115. Press RESET

//   116. Press START

//   117. Verify CHECK STOP & MAR CHK are on and MAR = 1/C/C/C/C

//   118. Press RESET

//   119. Press START

//

//==============================================================================

Sample Output – CS01

IBM 1620 Jr. SPS Assembler (v0.99)    Source: CS01_Check_Stops.sps    Assembled: 8/26/2020 @ 08:05

1                              *=====================================================================

2                              *

3                              * CHECK STOPS DIAGNOSTIC

4                              *

5                              * USAGE - TESTS ALL COMBINATION OF CHECK STOPS IN IBM 1620 JR.

6                              *

7                              * ALGORITHM - TEST ALL POSSIBLE NON-NUMERIC DIGITS IN ALL POSITIONS

8                              *             OF MEMORY ADDRESS REGISTER.

9                              *

10                              *             TEST AN UNDEFINED OP CODE.

11                              *

12                              *             TEST AN INVALID OP CODE.

13                              *

14                              *             TEST AN INSTRUCTION AT AN ODD ADDRESS.

15                              *

16                              *             TEST ACCESS TO MEMORY BEYOND 59999.

17                              *

18                              *             TEST A READ ALPHAMERIC INSTRUCTION TO AN EVEN ADDRESS.

19                              *

20                              *             TEST A WRITE ALPHAMERIC INSTRUCTION FROM AN EVEN ADDRESS  .

21                              *

22                              * NOTE - DIGIT COMBINATIONS 0X0B, 0X0D, 0X0E, AND 0X0F MUST BE

23                              *        MANUALLY ENTERED IN THE CHECK STOPS CMEM FILE.

24                              *

25                              *=====================================================================

26

27  00402 34 00000 00102        START  RCTY

28  00414 39 01447 00100               WATY  TITLE

29  00426 34 00000 00102               RCTY

30  00438 34 00000 00102               RCTY

31

32                              * MAR CHECKS

33  00450 39 01493 00100        MAR    WATY  MART

34  00462 34 00000 00102               RCTY

35

36                              * RECORD MARK (0XA) IN ALL POSITIONS

37  00474 39 01593 00100               WATY  MAR10

38  00486 34 00000 00102               RCTY

39

40  00498 330000                       DSC   6,330000

41  00504 @                            DSC   1,@

42  00505 00000                        DSC   5,00000

43

44  00510 33000                        DSC   5,33000

45  00515 @                            DSC   1,@

46  00516 000000                       DSC   6,000000

47

48  00522 3300                         DSC   4,3300

49  00526 @                            DSC   1,@

50  00527 0000000                      DSC   7,0000000

51

52  00534 330                          DSC   3,330

53  00537 @                            DSC   1,@

54  00538 00000000                     DSC   8,00000000

55

56  00546 33                           DSC   2,33

57  00548 @                            DSC   1,@

58  00549 000000000                    DSC   9,000000000

59

60                              * RECORD GROUP MARK (0XB) IN ALL POSITIONS

61  00558 39 01629 00100               WATY  MAR11

62  00570 34 00000 00102               RCTY

63

64  00582 33 00001 00000               CF    00001

65  00594 33 00010 00000               CF    00010

66  00606 33 00100 00000               CF    00100

67  00618 33 01000 00000               CF    01000

68  00630 33 10000 00000               CF    10000

69

70                              * NUMERIC BLANK (0XC) IN ALL POSITIONS

71  00642 39 01669 00100               WATY  MAR12

72  00654 34 00000 00102               RCTY

73

74  00666 330000                       DSC   6,330000

75  00672 ~                            DNB   1

76  00673 00000                        DSC   5,00000

77

78  00678 33000                        DSC   5,33000

79  00683 ~                            DNB   1

80  00684 000000                       DSC   6,000000

81

82  00690 3300                         DSC   4,3300

83  00694 ~                            DNB   1

84  00695 0000000                      DSC   7,0000000

85

86  00702 330                          DSC   3,330

87  00705 ~                            DNB   1

88  00706 00000000                     DSC   8,00000000

89

90  00714 33                           DSC   2,33

91  00716 ~                            DNB   1

92  00717 000000000                    DSC   9,000000000

93

94                              * INVALID (0XD) IN ALL POSITIONS

95  00726 39 01705 00100               WATY  MAR13

96  00738 34 00000 00102               RCTY

97

98  00750 33 00001 00000               CF    00001

99  00762 33 00010 00000               CF    00010

100  00774 33 00100 00000               CF    00100

101  00786 33 01000 00000               CF    01000

102  00798 33 10000 00000               CF    10000

103

104                              * INVALID (0XE) IN ALL POSITIONS

105  00810 39 01745 00100               WATY  MAR14

106  00822 34 00000 00102               RCTY

107

108  00834 33 00001 00000               CF    00001

109  00846 33 00010 00000               CF    00010

110  00858 33 00100 00000               CF    00100

111  00870 33 01000 00000               CF    01000

112  00882 33 10000 00000               CF    10000

113

114                              * GROUP MARK (0XF) IN ALL POSITIONS

115  00894 39 01785 00100               WATY  MAR15

116  00906 34 00000 00102               RCTY

117

118  00918 33 00001 00000               CF    00001

119  00930 33 00010 00000               CF    00010

120  00942 33 00100 00000               CF    00100

121  00954 33 01000 00000               CF    01000

122  00966 33 10000 00000               CF    10000

123

124  00978 39 01829 00100        INST   WATY  INSTT

125  00990 34 00000 00102               RCTY

126

127  01002 39 01895 00100               WATY  INSTB

128  01014 34 00000 00102               RCTY

129

130  01026 770000000000                 DSC   12,770000000000

131

132  01038 39 01945 00100               WATY  INSTN

133  01050 34 00000 00102               RCTY

134

135  01062 010000000000                 DSC   12,010000000000

136

137  01074 100000000000                 DSC   12,100000000000

138

139  01086 39 02001 00100               WATY  INSTU

140  01098 34 00000 00102               RCTY

141

142  01110 26 00011 02375               TF    11,IMEM+11

143  01122 49 08421 00000               B     08421

144

145  01134 26 00011 02363        MEM    TF    11,ISTRT+11

146  01146 39 02061 00100               WATY  MEMT

147  01158 34 00000 00102               RCTY

148

149  01170 39 02121 00100               WATY  MEMH

150  01182 34 00000 00102               RCTY

151

152  01194 33 60000 00000               CF    60000

153  01206 33 99999 00000               CF    99999

154

155  01218 39 02163 00100        IO     WATY  IOT

156  01230 34 00000 00102               RCTY

157

158  01242 39 02227 00100               WATY  IOR

159  01254 34 00000 00102               RCTY

160

161  01266 34 00000 00101               SPTY

162  01278 34 00000 00101               SPTY

163  01290 34 00000 00101               SPTY

164  01302 34 00000 00101               SPTY

165  01314 37 10000 00100               RATY  10000

166  01326 34 00000 00102               RCTY

167

168  01338 39 02275 00100               WATY  IOW

169  01350 34 00000 00102               RCTY

170

171  01362 39 10000 00100               WATY  10000

172

173  01374 26 00011 02363        END    TF    11,ISTRT+11

174  01386 34 00000 00102               RCTY

175  01398 39 02327 00100               WATY  ENDT

176  01410 34 00000 00102               RCTY

177  01422 48 00000 00000               H

178  01434 49 00402 00000               B     START

179

180                              *

181                              * DATA

182                              *

_

183  01447 434845..71040@        TITLE  DAC  23,CHECK STOP TEST (CS01)@

_

184  01493 544159..56550@        MART   DAC  50,MAR CHECKS - NON-NUMERIC DIGITS IN EVERY POSITION@

_

185  01593 000020..75040@        MAR10  DAC  18,  - DIGIT 8-2 (5)@

_

186  01629 000020..75040@        MAR11  DAC  20,  - DIGIT 8-2-1 (5)@

_

187  01669 000020..75040@        MAR12  DAC  18,  - DIGIT 8-4 (5)@

_

188  01705 000020..75040@        MAR13  DAC  20,  - DIGIT 8-4-1 (5)@

_

189  01745 000020..75040@        MAR14  DAC  20,  - DIGIT 8-4-2 (5)@

_

190  01785 000020..75040@        MAR15  DAC  22,  - DIGIT 8-4-2-1 (5)@

_

191  01829 565743..55620@        INSTT  DAC  33,OPCODE CHECKS - BAD INSTRUCTIONS@

_

192  01895 000020..71040@        INSTB  DAC  25,  - UNDEFINED OPCODE (1)@

_

193  01945 000020..72040@        INSTN  DAC  28,  - NON-NUMERIC OPCODES (2)@

_

194  02001 000020..71040@        INSTU  DAC  30,  - UNALIGNED INSTRUCTION (1)@

_

195  02061 544554..45620@        MEMT   DAC  30,MEMORY CHECKS - BAD ADDRESSES@

_

196  02121 000020..72040@        MEMH   DAC  21,  - OUT-OF-RANGE (2)@

_

197  02163 495600..63450@        IOT    DAC  32,IO CHECK - UNALIGNED READ/WRITE@

_

198  02227 000020..71040@        IOR    DAC  24,  - READ ALPHAMERIC (1)@

_

199  02275 000020..04000@        IOW    DAC  26,  - WRITE ALPHAMERIC (1)@

_

200  02327 455544..63620@        ENDT   DAC  13,END OF TESTS@

201

_

202  02352 49 00402 00000        ISTRT  B    START,,0

_

203  02364 49 01134 00000        IMEM   B    MEM,,0

204

205  00402                              DEND START

Symbol Cross-Reference Table

============================

Symbol   Addr.  Type    Defined  References

-------  -----  ------  -------  -------------------------------------------------------------

END      01374  <inst>      173

ENDT     02327  DAC         200      175

IMEM     02364  <inst>      203      142

INST     00978  <inst>      124

INSTB    01895  DAC         192      127

INSTN    01945  DAC         193      132

INSTT    01829  DAC         191      124

INSTU    02001  DAC         194      139

IO       01218  <inst>      155

IOR      02227  DAC         198      158

IOT      02163  DAC         197      155

IOW      02275  DAC         199      168

ISTRT    02352  <inst>      202      145      173

MAR      00450  <inst>       33

MAR10    01593  DAC         185       37

MAR11    01629  DAC         186       61

MAR12    01669  DAC         187       71

MAR13    01705  DAC         188       95

MAR14    01745  DAC         189      105

MAR15    01785  DAC         190      115

MART     01493  DAC         184       33

MEM      01134  <inst>      145      203

MEMH     02121  DAC         196      149

MEMT     02061  DAC         195      146

START    00402  <inst>       27      205      178      202      205

TITLE    01447  DAC         183       28

DX05 - Core Storage 20K Diagnostic

//==============================================================================

//

// DX05L - Core Storage L 20K Diagnostic

//

// Program Switch settings:

//

//   PS1:   ON - Bypass error type out

//         OFF - Type out routine number on error

//   PS2:   ON - Loop in routine

//         OFF - Continue to next routine

//   PS3:  not used

//   PS4:   ON - Repeat test DX05L

//         OFF - Run test DX05L once

//

// Check switches settings:

//

//   DISK I/O - STOP

//   PARITY   - STOP

//   I/O      - STOP

//   O'FLOW   - STOP

//

// Start addresses:

//

//   05052 - Full test

//

// Directions:

//

//   1. Load DX05L diagnostic

//   2. Press START

//

//==============================================================================

//==============================================================================

//

// DX05H - Core Storage H 20K Diagnostic

//

// Program Switch settings:

//

//   PS1:   ON - Bypass error type out

//         OFF - Type out routine number on error

//   PS2:   ON - Loop in routine

//         OFF - Continue to next routine

//   PS3:  not used

//   PS4:   ON - Repeat test DX05H

//         OFF - Run test DX05H once

//

// Check switches settings:

//

//   DISK I/O - STOP

//   PARITY   - STOP

//   I/O      - STOP

//   O'FLOW   - STOP

//

// Start addresses:

//

//   00402 - Full test

//

// Directions:

//

//   1. Load DX05H diagnostic

//   2. Press START

//

//==============================================================================

Sample Output – DX05

CU04 - Additional Core Diagnostic

//==============================================================================

//

// CU04 - Additional Core Diagnostic

//

// Program Switch settings:

//

//   PS1:   ON - Bypass error type out

//         OFF - Type out routine number on error

//   PS2:   ON - Loop in routine

//         OFF - Continue to next routine

//   PS3:   ON - Stop on error

//         OFF - Do not stop on error, continue

//   PS4:  not used

//

// Check switches settings:

//

//   DISK I/O - STOP

//   PARITY   - STOP

//   I/O      - STOP

//   O'FLOW   - PROGRAM

//

// Start addresses:

//

//   00828 - Full test

//

// Directions:

//

//   1. Load CU04 diagnostic

//   2. Press START

//   3. Press START

//   4. Type <flag 6> 0

//   5. Press RELEASE-START

//

//==============================================================================

Sample Output – CU04

// CU04 - Additional Core Diagnostic (IBM manual not available)

00000: 04 09 00 00 08 02 08 0A 00 00  00 00 00 00 00 00 00 00 00 00  // "I  #       "

00100: 00 00 00 00 00 00 00 00 00 00  00 00 01 00 02 00 03 00 04 00  // "       +-  "

00120: 00 00 02 00 04 00 06 00 08 00  00 00 03 00 06 00 09 00 02 01  // " -        /"

00140: 00 00 04 00 08 00 02 01 06 01  00 00 05 00 00 01 05 01 00 02  // "   /   - J "

00160: 00 00 06 00 02 01 08 01 04 02  00 00 07 00 04 01 01 02 08 02  // "  / B  0A  "

00180: 00 00 08 00 06 01 04 02 02 03  00 00 09 00 08 01 07 02 06 03  // "   B,    2T"

00200: 00 00 00 00 00 00 00 00 00 00  05 00 06 00 07 00 08 00 09 00  // "      - 0  "

00220: 00 01 02 01 04 01 06 01 08 01  05 01 08 01 01 02 04 02 07 02  // " /A   J  B2"

00240: 00 02 04 02 08 02 02 03 06 03  05 02 00 03 05 03 00 04 05 04  // " B ,T K.L)M"

00260: 00 03 06 03 02 04 08 04 04 05  05 03 02 04 09 04 06 05 03 06  // ".T( E L( V "

00280: 00 04 08 04 06 05 04 06 02 07  05 04 04 05 03 06 02 07 01 08  // ") VF  ME   "

00300: 00 01 02 03 04 05 06 07 08 09  01 02 03 04 05 06 07 08 09 10  // " ,EX   @O8 "

00320: 02 03 04 05 06 07 08 09 10 11  03 04 05 06 07 08 09 10 11 12  // ",EX   @O8  "

00340: 04 05 06 07 08 09 10 11 12 13  05 06 07 08 09 10 11 12 13 14  // "EX  , O8  @"

00360: 06 07 08 09 10 11 12 13 14 15  07 08 09 10 11 12 13 14 15 16  // "X  ,E 8  @O"

00380: 08 09 10 11 12 13 14 15 16 17  09 10 11 12 13 14 15 16 17 18  // "  ,EX   @O8"

00400: 0A 00 00 00 00 00 00 00 05 02  04 05 06 08 00 00 04 09 05 05  // "    K EY IN"

00420: 00 00 05 04 04 05 05 04 00 00  04 03 05 06 05 05 06 02 06 03  // " MEM  CONST"

00440: 06 02 00 00 05 09 04 05 05 03  04 05 04 01 06 02 04 05 00 03  // "S REL EASE."

00460: 00 00 06 02 06 03 04 01 05 09  06 03 00 03 00 00 00 0A 00 00  // " STAR T. # "

00480: 00 00 00 00 00 00 00 00 00 00  00 00 06 02 06 03 04 01 05 09  // "       STAR"

00500: 06 03 00 00 05 09 05 06 06 04  06 03 04 09 05 05 04 05 06 02  // "T ROU TINES"

00520: 00 03 00 00 04 05 06 03 05 06  06 02 00 00 04 06 05 06 05 03  // ". ETO S FOL"

00540: 05 03 05 06 06 06 00 03 00 00  00 0A 00 00 00 00 00 00 00 00  // "LOW.  #    "

00560: 00 00 00 00 06 02 06 06 00 00  07 01 00 00 05 06 05 05 00 00  // "  SW  1 ON "

00580: 00 0A 06 02 06 06 00 00 07 01  00 00 05 06 04 06 04 06 00 00  // "#SW 1  OFF "

00600: 00 0A 06 02 06 06 00 00 07 02  00 00 05 06 05 05 00 00 00 0A  // "#SW 2  ON #"

00620: 06 02 06 06 00 00 07 02 00 00  05 06 04 06 04 06 00 00 00 0A  // "SW 2  OFF #"

00640: 06 02 06 06 00 00 07 03 00 00  05 06 05 05 00 00 00 0A 06 02  // "SW 3  ON #S"

00660: 06 06 00 00 07 03 00 00 05 06  04 06 04 06 00 00 00 0A 06 02  // "W 3 O FF #S"

00680: 06 06 00 00 07 04 00 00 05 06  05 05 00 00 00 0A 06 02 06 06  // "W 4 O N #SW"

00700: 00 00 07 04 00 00 05 06 04 06  04 06 00 00 00 0A 06 02 04 05  // " 4 OF F #SE"

00720: 06 03 00 00 06 02 06 06 06 02  00 00 04 06 05 06 05 09 00 00  // "T SWS  FOR "

00740: 04 01 04 04 04 04 04 09 06 03  04 09 05 06 05 05 04 01 05 03  // "ADDIT IONAL"

00760: 00 00 05 04 04 05 05 04 05 06  05 09 06 08 00 00 06 03 04 05  // " MEMO RY TE"

00780: 06 02 06 03 00 03 00 00 06 03  04 08 04 05 05 05 00 00 06 02  // "ST. T HEN S"

00800: 06 03 04 01 05 09 06 03 00 03  00 00 00 0A 00 00 00 00 00 00  // "TART.  #   "

00820: 00 00 00 00 00 00 00 00 04 06  00 00 08 05 02 00 00 01 00 00  // "    F   -  "

00840: 04 07 00 00 08 07 06 00 00 01  00 00 03 09 00 00 05 06 05 00  // "G        O-"

00860: 00 01 00 00 04 09 00 00 08 08  08 00 00 00 00 00 03 09 00 00  // "  I        "

00880: 05 08 03 00 00 01 00 00 04 06  00 00 09 01 02 00 00 02 00 00  // "Q   F   -  "

00900: 04 07 00 00 09 03 06 00 00 02  00 00 03 09 00 00 06 00 03 00  // "G          "

00920: 00 01 00 00 04 09 00 00 09 04  08 00 00 00 00 00 03 09 00 00  // "  I        "

00940: 06 02 01 00 00 01 00 00 04 06  00 00 09 07 02 00 00 03 00 00  // "S+  F   -. "

00960: 04 07 00 00 09 09 06 00 00 03  00 00 03 09 00 00 06 04 01 00  // "G   .    U+"

00980: 00 01 00 00 04 09 00 01 00 00  08 00 00 00 00 00 03 09 00 00  // "  I        "

01000: 06 05 09 00 00 01 00 00 04 06  00 01 00 03 02 00 00 04 00 00  // "V   F  .-) "

01020: 04 07 00 01 00 05 06 00 00 04  00 00 03 09 00 00 06 07 09 00  // "G   )    X "

01040: 00 01 00 00 04 09 00 01 00 06  08 00 00 00 00 00 03 09 00 00  // "  I        "

01060: 06 09 07 00 00 01 00 00 03 09  00 00 07 01 07 00 00 01 00 00  // "Z0     10  "

01080: 04 08 00 00 00 00 00 00 00 00  00 00 03 04 00 00 00 00 00 00  // "H      @   "

01100: 00 01 00 02 03 09 00 00 04 09  03 00 00 01 00 00 04 09 00 01  // "    I    I "

01120: 01 04 00 00 00 00 00 00 00 00  00 00 00 00 00 00 00 00 00 00  // "*          "

01140: 03 09 00 00 04 00 09 00 00 01  00 00 03 06 00 01 01 03 03 00  // "         $ "

01160: 00 01 00 00 04 09 00 01 03 00  08 00 00 00 00 00 00 00 00 00  // "  I        "

01180: 00 00 00 00 00 00 00 00 10 00  00 03 00 04 01 00 01 03 01 04  // "      .)+$*"

01200: 02 00 02 01 02 03 02 04 03 03  03 04 04 01 04 02 04 03 04 04  // "-/,(= @ABCD"

01220: 04 05 04 06 04 07 04 08 04 09  05 01 05 02 05 03 05 04 05 05  // "EFGHI JKLMN"

01240: 05 06 05 07 05 08 05 09 06 01  06 02 06 03 06 04 06 05 06 06  // "OPQR  STUVW"

01260: 06 07 06 08 06 09 07 00 07 01  07 02 07 03 07 04 07 05 07 06  // "XYZ01 23456"

01280: 07 07 07 08 07 09 06 09 00 0A  00 00 00 00 00 00 00 00 00 00  // "789Z#      "

01300: 00 00 00 00 00 00 00 00 03 01  02 09 09 05 00 00 01 01 08 08  // "           "

01320: 02 04 00 01 02 08 07 03 00 00  04 09 04 07 00 01 03 08 00 00  // "(  3  IG   "

01340: 01 02 00 00 04 09 00 01 04 07  06 00 00 00 00 00 00 00 00 00  // "  I G      "

01380: 04 06 00 01 04 05 02 00 00 01  00 00 03 09 00 01 04 06 05 00  // "F E-     F-"

01400: 00 01 00 00 03 04 00 00 00 00  00 00 00 01 00 02 03 09 00 01  // "  @        "

01420: 01 08 09 00 00 01 00 00 03 04  00 00 00 00 00 00 00 01 00 02  // "    @      "

01440: 03 09 02 09 09 05 01 00 00 01  00 00 04 07 00 01 04 07 06 00  // "   +   G G "

01460: 00 03 00 00 04 08 07 00 07 00  07 03 00 00 00 0A 04 06 00 01  // ". H00 3 #F "

01480: 03 00 08 00 00 02 00 00 04 09  00 01 05 04 08 00 00 00 00 00  // "    I  M   "

01540: 00 00 00 00 00 00 00 00 03 01  00 09 09 05 00 02 09 09 05 00  // "          -"

01560: 02 04 03 00 00 04 09 01 00 00  04 09 04 07 00 01 06 03 02 00  // "( )   IG T-"

01580: 01 02 00 00 04 09 00 01 07 02  08 00 00 00 00 00 00 00 00 00  // "  I 2      "

01620: 00 00 00 00 00 00 00 00 00 00  00 00 04 06 00 01 07 00 04 00  // "       F 0 "

01640: 00 01 00 00 03 09 00 01 07 01  07 00 00 01 00 00 03 04 00 00  // "    1 0  @ "

01660: 00 00 00 00 00 01 00 02 03 09  02 09 09 05 01 00 00 01 00 00  // "        +  "

01680: 03 04 00 00 00 00 00 00 00 01  00 02 03 09 00 09 09 05 01 00  // "@         +"

01700: 00 01 00 00 04 07 00 01 07 02  08 00 00 03 00 00 04 08 07 00  // "  G 2  . H0"

01720: 07 00 07 04 00 00 00 0A 04 06  00 01 05 04 08 00 00 02 00 00  // "04 #F  M   "

01740: 04 09 00 01 08 07 02 00 00 00  00 00 00 00 00 00 00 00 00 00  // "I  -       "

01760: 00 00 00 00 00 00 00 00 00 00  00 00 00 00 00 00 04 03 05 06  // "         CO"

01780: 05 05 06 02 06 03 06 02 00 00  04 09 05 05 04 03 05 06 05 09  // "NSTS  INCOR"

01800: 05 09 00 03 00 00 05 02 04 05  06 08 00 00 04 09 05 05 00 00  // "R. KE Y IN "

01820: 04 03 05 06 05 09 05 09 00 00  04 03 05 06 05 05 06 02 06 03  // "CORR  CONST"

01840: 06 06 00 00 04 01 05 05 04 04  00 00 06 02 06 03 04 01 05 09  // "W AND  STAR"

01860: 04 05 06 03 00 03 00 00 00 0A  00 00 01 04 00 01 01 03 04 00  // "ET. #  * $ "

01880: 00 00 14 00 04 06 00 03 00 09  06 00 01 02 00 00 01 04 00 01  // "  F.     * "

01900: 01 03 04 00 00 00 16 00 04 06  00 02 00 08 08 00 01 02 00 00  // "$   F      "

01920: 03 09 00 01 07 07 07 00 00 01  00 00 04 09 00 01 01 04 00 00  // "  70   I * "

01960: 00 00 00 00 00 00 00 00 16 09  07 09 07 08 07 07 07 06 07 05  // "    Z 98765"

01980: 07 04 07 03 07 02 07 01 07 00  06 09 06 08 06 07 06 06 06 05  // "43210 ZYXWV"

02000: 06 04 06 03 06 02 06 01 05 09  05 08 05 07 05 06 05 05 05 04  // "UTS R QPONM"

02020: 05 03 05 02 05 01 04 09 04 08  04 07 04 06 04 05 04 04 04 03  // "LKJIH GFEDC"

02040: 04 02 04 01 03 04 03 03 02 04  02 03 02 01 02 00 01 04 01 03  // "BA@=( ,/-*$"

02060: 01 00 00 04 00 00 00 03 00 0A  00 00 00 00 00 00 00 00 00 00  // "+) .#      "

02080: 00 00 00 00 00 00 00 00 03 01  04 09 09 05 00 00 01 09 06 08  // "      I   Y"

02100: 02 04 00 02 00 06 07 05 00 00  04 09 04 07 00 02 01 06 00 00  // "(  5  IG   "

02120: 01 02 00 00 04 09 00 02 02 05  06 00 00 00 00 00 00 00 00 00  // "  I        "

02160: 04 06 00 02 02 03 02 00 00 01  00 00 03 09 00 02 02 04 05 00  // "F ,-     (-"

02180: 00 01 00 00 03 04 00 00 00 00  00 00 00 01 00 02 03 09 00 01  // "  @        "

02200: 09 06 09 00 00 01 00 00 03 04  00 00 00 00 00 00 00 01 00 02  // "    @      "

02220: 03 09 04 09 09 05 01 00 00 01  00 00 04 07 00 02 02 05 06 00  // " I +   G   "

02240: 00 03 00 00 04 08 07 00 07 00  07 06 00 00 00 0A 04 06 00 02  // ". H00 6 #F "

02260: 00 08 08 00 00 02 00 00 04 09  00 02 03 00 04 00 00 00 00 00  // "    I      "

02300: 00 00 00 00 03 01 00 09 09 05  00 04 09 09 05 00 02 04 05 00  // "      ) -(-"

02320: 00 04 09 01 00 00 04 09 04 07  00 02 03 07 06 00 01 02 00 00  // ")  IG      "

02340: 04 09 00 02 04 07 02 00 00 00  00 00 00 00 00 00 00 00 00 00  // "I G-       "

02360: 00 00 00 00 00 00 00 00 00 00  00 00 00 00 00 00 04 06 00 02  // "         F "

02380: 04 04 08 00 00 01 00 00 03 09  00 02 04 06 01 00 00 01 00 00  // "D      F+  "

02400: 03 04 00 00 00 00 00 00 00 01  00 02 03 09 04 09 09 05 01 00  // "@       I +"

02420: 00 01 00 00 03 04 00 00 00 00  00 00 00 01 00 02 03 09 00 09  // "  @        "

02440: 09 05 01 00 00 01 00 00 04 07  00 02 04 07 02 00 00 03 00 00  // " +  G  G-. "

02460: 04 08 07 00 07 00 07 03 00 00  00 0A 04 06 00 02 03 00 04 00  // "H003  #F   "

02480: 00 02 00 00 04 09 00 02 06 04  00 00 00 00 00 00 00 00 00 00  // "  I U      "

02520: 14 01 04 02 04 03 04 04 04 05  04 06 04 07 04 08 04 09 05 01  // "ABCDE FGHIJ"

02540: 05 02 05 03 05 04 05 05 05 06  05 07 05 08 05 09 06 01 06 02  // "KLMNO PQR S"

02560: 06 03 06 04 06 05 06 06 06 07  06 08 06 09 00 00 00 03 00 04  // "TUVWX YZ .)"

02580: 01 00 01 03 01 04 02 00 02 01  02 03 02 04 03 03 03 04 07 00  // "+$*-/ ,(=@0"

02600: 07 01 07 02 07 03 07 04 07 05  07 06 07 07 07 08 07 09 06 09  // "12345 6789Z"

02620: 00 0A 00 00 00 00 00 00 00 00  00 00 00 00 00 00 00 00 00 00  // "#          "

02640: 03 01 02 09 08 05 00 00 02 05  02 00 03 01 04 09 09 05 00 02  // "      - I  "

02660: 09 08 05 00 02 04 05 00 00 04  09 02 09 09 04 09 04 07 00 02  // " -(-)   IG "

02680: 07 02 04 00 01 02 00 00 04 09  00 02 08 02 00 00 00 00 00 00  // "2   I      "

02720: 00 00 00 00 04 06 00 02 07 09  06 00 00 01 00 00 03 09 00 02  // "  F 9      "

02740: 08 00 09 00 00 01 00 00 03 04  00 00 00 00 00 00 00 01 00 02  // "    @      "

02760: 03 09 02 09 08 05 01 00 00 01  00 00 03 04 00 00 00 00 00 00  // "   +   @   "

02780: 00 01 00 02 03 09 04 09 09 05  01 00 00 01 00 00 04 07 00 02  // "   I  +  G "

02800: 08 02 00 00 00 03 00 00 04 08  07 00 07 00 07 08 00 00 00 0A  // "  . H 008 #"

02820: 04 06 00 02 06 04 00 00 00 02  00 00 04 09 00 02 08 06 08 00  // "F U    I   "

02860: 00 00 00 00 00 00 00 00 03 01  02 09 09 05 00 04 09 09 05 00  // "        ) -"

02880: 02 04 03 00 00 04 09 05 00 00  04 09 04 07 00 02 09 04 00 00  // "( )   IG   "

02900: 01 02 00 00 04 09 00 03 00 03  06 00 00 00 00 00 00 00 00 00  // "  I..      "

02940: 04 06 00 03 00 01 02 00 00 01  00 00 03 09 00 03 00 02 03 00  // "F. -    .  "

02960: 00 01 00 00 03 04 00 00 00 00  00 00 00 01 00 02 03 09 04 09  // "  @       I"

02980: 09 05 01 00 00 01 00 00 03 04  00 00 00 00 00 00 00 01 00 02  // " +  @      "

03000: 03 09 02 09 09 05 01 00 00 01  00 00 04 07 00 03 00 03 06 00  // "   +   G.. "

03020: 00 03 00 00 04 08 07 00 07 00  07 09 00 00 00 0A 04 06 00 02  // ". H00 9 #F "

03040: 08 06 08 00 00 02 00 00 04 09  00 03 00 09 06 00 00 00 00 00  // "    I .    "

03080: 00 00 00 00 00 00 00 00 00 00  00 00 00 00 10 00 04 06 00 03  // "         F."

03100: 01 00 08 00 01 04 00 00 01 01  00 03 00 09 05 00 00 00 10 01  // "+ *   . -  "

03120: 04 07 00 01 03 00 08 00 01 04  00 00 04 09 00 03 01 06 08 00  // "G   *  I.  "

03160: 00 00 00 00 00 00 00 00 04 01  03 09 09 05 02 00 00 03 00 00  // "    A   -. "

03180: 03 04 00 00 00 00 00 00 00 01  00 02 03 05 03 09 09 07 06 00  // "@          "

03200: 00 01 00 00 03 04 00 00 00 00  00 00 00 01 00 01 04 09 03 09  // "  @      I "

03220: 09 05 02 00 00 00 00 00 04 09  00 03 02 04 00 00 00 00 00 00  // " -  I .(   "

03240: 04 01 02 05 08 05 01 00 00 03  00 00 03 04 00 00 00 00 00 00  // "A  +.  @   "

03260: 00 01 00 02 03 09 02 05 08 05  01 00 00 01 00 00 04 09 00 03  // "      +  I."

03280: 02 08 08 00 00 00 00 00 01 04  00 01 01 03 04 00 00 00 14 00  // "    *  $   "

03300: 04 06 00 03 06 08 04 00 01 02  00 00 01 04 00 01 01 03 04 00  // "F.Y    * $ "

03320: 00 00 16 00 04 06 00 03 03 06  00 00 01 02 00 00 03 09 00 01  // "  F.       "

03340: 07 07 07 00 00 01 00 00 04 09  00 01 01 04 00 00 00 00 00 00  // "70  I  *   "

03360: 04 01 05 09 09 05 02 00 00 03  00 00 03 04 00 00 00 00 00 00  // "AR -.  @   "

03380: 00 01 00 02 03 05 05 09 09 07  06 00 00 01 00 00 03 04 00 00  // "   R     @ "

03400: 00 00 00 00 00 01 00 01 04 09  05 09 09 05 02 00 00 00 00 00  // "    I R -  "

03420: 04 09 00 03 04 03 02 00 00 00  00 00 04 01 04 05 08 05 01 00  // "I.C-   AE +"

03440: 00 03 00 00 03 04 00 00 00 00  00 00 00 01 00 02 03 09 04 05  // ". @       E"

03460: 08 05 01 00 00 01 00 00 04 09  00 03 06 08 04 00 00 00 00 00  // " +  I .Y   "

03480: 06 03 04 05 06 02 06 03 00 00  05 09 05 06 06 04 06 03 04 09  // "TEST  ROUTI"

03500: 05 05 04 05 06 02 00 00 04 03  05 06 05 04 05 07 05 03 04 05  // "NES C OMPLE"

03520: 06 03 04 05 04 04 00 03 00 00  04 09 04 06 00 00 06 02 06 06  // "TED.  IF SW"

03540: 00 00 07 01 00 00 05 06 04 06  04 06 00 00 04 01 05 05 04 04  // " 1 OF F AND"

03560: 00 00 05 05 05 06 00 00 05 09  05 06 06 04 06 03 04 09 05 05  // " NO R OUTIN"

03580: 04 05 00 00 05 05 05 06 06 02  00 00 06 03 06 08 05 07 04 05  // "E NOS  TYPE"

03600: 04 04 00 00 05 06 06 04 06 03  02 03 00 00 05 04 04 01 04 03  // "D OUT , MAC"

03620: 04 08 04 09 05 05 04 05 00 00  05 07 04 05 05 09 04 06 05 06  // "HINE  PERFO"

03640: 05 09 05 04 04 05 04 04 00 00  06 03 04 05 06 02 06 03 06 02  // "RMED  TESTS"

03660: 00 00 05 07 05 09 05 06 05 07  04 05 05 09 05 03 06 08 00 03  // " PROP ERLY."

03680: 00 00 00 0A 03 04 00 00 00 00  00 00 00 01 00 02 03 09 00 03  // " #@       ."

03700: 04 08 01 00 00 01 00 00 04 08  00 00 00 00 00 00 00 00 00 00  // "H+  H      "

03720: 00 00 00 00 00 00 00 00 00 00  00 0A 00 00 00 00 00 00 00 00  // "      #    "

25840: 00 00 00 00 00 00 00 00 00 00  04 01 04 02 04 03 04 04 04 05  // "      ABCDE"

25860: 04 06 04 07 04 08 04 09 05 01  05 02 05 03 05 04 05 05 05 06  // "FGHIJ KLMNO"

25880: 05 07 05 08 05 09 06 02 06 03  06 04 06 05 06 06 06 07 06 08  // "PQRST UVWXY"

25900: 06 09 00 03 00 04 01 00 01 03  01 04 02 00 02 01 02 03 02 04  // "Z.)+$ *-/,("

25920: 03 03 03 04 07 00 07 01 07 02  07 03 07 04 07 05 07 06 07 07  // "=@012 34567"

25940: 07 08 07 09 00 0A 00 00 00 00  00 00 00 00 00 00 00 00 00 00  // "89#        "

39940: 00 00 00 00 00 00 00 00 00 00  00 00 03 08 03 09 09 09 04 00  // "           "

39960: 00 01 00 00 04 09 00 03 02 02  08 0A 00 00 00 00 03 09 09 07  // "  I.  #    "

39980: 06 00 01 02 03 04 05 06 07 08  09 0A 11 12 03 09 09 09 08 09  // "  @O8 #    "

40000: 04 00 00 00 04 0A 00 00 00 00  00 00 00 00 00 00 00 00 00 00  // "  #        "

45840: 00 00 00 00 00 00 00 00 00 00  07 00 07 01 07 02 07 03 07 04  // "      01234"

45860: 07 05 07 06 07 07 07 08 07 09  00 03 00 04 01 00 01 03 01 04  // "56789 .)+$*"

45880: 02 00 02 01 02 03 02 04 03 03  03 04 04 01 04 02 04 03 04 04  // "-/,(= @ABCD"

45900: 04 05 04 06 04 07 04 08 04 09  05 01 05 02 05 03 05 04 05 05  // "EFGHI JKLMN"

45920: 05 06 05 07 05 08 05 09 06 02  06 03 06 04 06 05 06 06 06 07  // "OPQRS TUVWX"

45940: 06 08 06 09 00 0A 00 00 00 00  00 00 00 00 00 00 00 00 00 00  // "YZ#        "

59940: 00 00 00 00 00 00 00 00 00 00  00 00 03 08 05 09 09 09 04 00  // "        R  "

59960: 00 01 00 00 04 09 00 03 04 02  00 0A 00 00 00 00 05 09 09 07  // "  I.B #  R "

59980: 06 00 01 02 03 04 05 06 07 08  09 0A 11 12 05 09 09 09 08 09  // "  @O8 # R  "

// End

DX03 – Console Typewriter Diagnostic

//==============================================================================

//

// DX03 – Console Typewriter Diagnostic

//

// Program Switch settings:

//

//   PS1:   ON - Bypass error type out

//         OFF - Type out routine number on error

//   PS2:   ON - Loop in routine

//         OFF - Continue to next routine

//   PS3:   ON - Stop on error

//         OFF - Do not stop on error, continue

//   PS4:   ON - Repeat test DX03

//         OFF - Run test DX03 once

//

// Check switches settings:

//

//   DISK I/O - STOP

//   PARITY   - STOP

//   I/O      - STOP

//   O'FLOW   - PROGRAM

//

// Start addresses:

//

//   00440 - Full test w/ title

//   00726 - Full test w/o title

//   01982 - Alphameric echo test

//   02256 - Numeric echo test

//

// Directions:

//

//   1. Load DX03 diagnostic

//   2. Tab stops are automatically set at 45 & 75

//   3. Press START

//   4. Press START

//   5. Press INSERT

//   6. Type 4901982

//   7. Press RELEASE

//   8. Press START

//   9. Type alphameric characters, end with record mark

//  10. Press RELEASE-START

//  11. Press INSERT

//  12. Type 4902256

//  13. Press RELEASE

//  14. Press START

//  15. Type numeric characters, end with record mark

//  16. Press RELEASE-START

//

//==============================================================================

Sample Output – DX03

IO02 - Card I/O Diagnostic

//==============================================================================

//

// IO02 - Card I/O Diagnostic

//

// Program Switch settings:

//

//   PS1:   ON - Bypass error type out

//         OFF - Type out routine number on error

//   PS2:   ON - Loop in routine

//         OFF - Continue to next routine

//   PS3:   ON - Stop on error

//         OFF - Do not stop on error, continue

//   PS4:   ON - Repeat test IO02

//         OFF - Run test IO02 once

//

// Check switches settings:

//

//   DISK I/O - STOP

//   PARITY   - STOP

//   I/O      - PROGRAM

//   O'FLOW   - PROGRAM

//

// Start addresses:

//

//   00828 - Full test

//   07152 - Punch alphameric ripple deck

//   02796 - Verify alphameric ripple deck

//   07188 - Punch numeric ripple deck

//   05304 - Verify numeric ripple deck

//

// Directions:

//

//   1. Load IO02 diagnostic

//   2. Press START

//   3. Press START

//   3. Press RELEASE

//   5. Press RESET

//   6. Press INSERT

//   7. Type 4907152

//   8. Press RELEASE-START

//   9. Verify that PUNCH NO FEED light is lit

//  10. Insert empty card deck into punch

//  11. Press START

//  12. Verify that READER NO FEED light is lit

//  13. Move card deck from punch to reader

//  14. Press RELEASE

//  15. Press RESET

//  16. Press INSERT

//  17. Type 4907188

//  18. Press RELEASE-START

//  19. Verify that PUNCH NO FEED light is lit

//  20. Insert empty card deck into punch

//  21. Press START

//  22. Verify that READER NO FEED light is lit

//  23. Move card deck from punch to reader

//

//==============================================================================

Sample Output – IO02

IO03 - Card I/O Reliability Diagnostic

//==============================================================================

//

// IO03 - Card I/O Reliability Diagnostic

//

// Program Switch settings:

//

//   PS1:   ON - Punch only

//         OFF - Read and punch

//   PS2:   ON - Read only

//         OFF - Read and punch

//   PS3:   ON - Don't print errors

//         OFF - Print errors

//   PS4:   ON - Delay constant

//         OFF - Delay changes

//

// Check switches settings:

//

//   DISK I/O - STOP

//   PARITY   - STOP

//   I/O      - PROGRAM

//   O'FLOW   - PROGRAM

//

// Start addresses:

//

//   00652 - Full test

//

// Directions:

//

//   1. Load IO03 diagnostic

//   2. Turn on PROGRAM SWITCH 1

//   3. Press START

//   4. Press START

//   5. Verify that PUNCH NO FEED light is lit

//   6. Insert empty card deck into punch

//   7. After ~ 1 minute press STOP

//   8. Remove cards from punch

//   9. Turn off PROGRAM SWITCH 1

//  10. Press RESET

//  11. Press INSERT

//  12. Type 4900652

//  13. Press RELEASE

//  14. Press START

//  15. Verify that READER NO FEED light is lit

//  16. Insert newly punched cards in reader

//  17. Verify that PUNCH NO FEED light is lit

//  18. Insert empty card deck into punch

//

//==============================================================================

Sample Output – IO03